| Subject Code 25ES1EE104 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

ELEMENTS OF ELECTRICAL AND ELECTRONICS ENGINEERING

(Common to ME & AE)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Define ideal and practical dependent sources | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | State average and RMS values of alternating quantity | 2M | CO2 | BL1 |
|  | List the types of DC generators. | 2M | CO3 | BL1 |
|  | List the types of earthing. | 2M | CO3 | BL1 |
|  | Draw or state the V-I characteristics of a Zener diode. | 2M | CO4 | BL2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - State and explain Kirchhoff’s Current Law (KCL) and Kirchhoff’s Voltage Law (KVL). | 5M | CO1 | BL2 |
|  | - Determine equivalent resistance between terminal A and B in the given circuit | 5M | CO1 | BL3 |
| OR |  |  |  |  |
| 2 | - Describe series and parallel combinations of resistive circuits. Derive the expressions for equivalent resistance in each case. | 5M | CO1 | BL2 |
|  | - Determine current flowing through 4Ω resistor in the given circuit using super position theorem. | 5M | CO1 | BL4 |
| UNIT-II |  |  |  |  |
| 3 | - Analyze a series RL circuit supplied by a sinusoidal voltage source. Derive the expression for impedance, current, and phase angle. | 5M | CO2 | BL2 |
|  | - Derive the relationship between line voltage and phase voltage in a three-phase star-connected system. | 5M | CO2 | BL3 |
| OR |  |  |  |  |
| 4 | - Define real power, reactive power, and apparent power. Explain the relationship among them using a power triangle. | 5M | CO2 | BL2 |
|  | - A series RLC circuit has: R=20Ω,L=0.15H,C=100μF connected to a 230V, 50Hzsource. Determine: Reactance’s, net reactance, impedance, circuit current and power factor. | 5M | CO2 | BL3 |
| UNIT-III |  |  |  |  |
| 5 | - Derive the EMF equation of a transformer and explain the significance of each term. | 5M | CO3 | BL2 |
|  | - A DC shunt motor takes: Supply voltage of 220V,armature current of 25A and armature resistance = 0.5Ω. Calculate: - Back EMF - Armature power developed | 5M | CO3 | BL4 |
| OR |  |  |  |  |
| 6 | - Explain the working principle of a DC generator with a neat sketch. | 5M | CO3 | BL2 |
|  | - Discuss the speed control methods of DC motor. | 5M | CO3 | BL3 |
| UNIT-IV |  |  |  |  |
| 7 | - Describe the construction, working principle, and applications of Switch Fuse Unit (SFU). | 5M | CO3 | BL2 |
|  | - Explain the effects of improper earthing in electrical systems. | 5M | CO3 | BL2 |
| OR |  |  |  |  |
| 8 | - Compare MCB and MCCB in the aspects of operation and application. | 5M | CO3 | BL4 |
|  | - Explain different types of electrical wiring systems used in domestic and industrial installations. | 5M | CO3 | BL2 |
| UNIT-V |  |  |  |  |
| 9 | - Draw and explain the V-I characteristics of a semiconductor diode. | 5M | CO4 | BL2 |
|  | - Describe the working of CE amplifier with neat circuit diagram and waveforms. | 5M | CO4 | BL2 |
| OR |  |  |  |  |
| 10 | - Explain the operation of a Full Wave Rectifier using centre-tapped transformer with neat waveforms. | 5M | CO4 | BL2 |
|  | - Explain fixed bias method of transistor biasing with circuit diagram. | 5M | CO4 | BL2 |

****