| Subject Code 25PC1EE102 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

Electrical Circuits-II

(EEE)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What are the minimum number of wattmeters required to measure three-phase power in a balanced three-phase circuit? Justify your answer. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | In a series RC circuit with R = 15 Ω and C = 20 µF, a DC voltage is suddenly applied at t = 0. Determine the time at which the current becomes 36.8% of its peak value. | 2M | CO-2 | BL-3 |
|  | Define driving point impedance, and what is its significance? | 2M | CO-3 | BL-2 |
|  | What are the conditions for reciprocity and conditions for symmetry for “Z” and “ABCD” Parameters | 2M | CO-3 | BL-2 |
|  | Define Fourier series and write the general trigonometric Fourier series representation of a periodic waveform. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Derive the relations between line voltage and phase voltage in a three-phase delta-connected system | 5M | CO-1 | BL-2 |
|  | - Two wattmeters are connected to measure power in a three-phase circuit. The readings of one of the wattmeter is 5kW when the load power factor is unity. If the power factor of the load is changed to 0.707 lagging without changing the total input power, calculate the reading of two wattmeters. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | A three-phase four-wire 208V, CBA system as shown in figure below has a star connected load with ZA=5∠00Ω, ZB=5∠300Ω and ZC=10∠-600Ω. Obtain the phase currents, line currents and current through neutral wire. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Derive the transient response of a series RC circuit with dc input. Sketch the variation of current through the capacitor. | 5M | CO-2 | BL-2 |
|  | - A series RL circuit shown in figure below has a constant voltage V applied at t=0. At what time does VR = VL. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | For a series R-L Circuit V=50V, R=15 Ω, and L=20 mH. At t=0, the switch is closed. Determine the current response in the circuit and show the voltages across the resistance and inductance. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Explain the properties of the driving point impedance of a passive RLC network. | 5M | CO-3 | BL-2 |
|  | - For the series RLC network having (R = 10Ω), (L = 2H), and (C=0.05F): - Derive the driving-point impedance function (Z(s)). - Determine the poles and zeros of the network function. - Comment on the nature of the poles and the stability of the network. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | The voltage V(s) of a network is given by Vs=3s(s+2)(s2+2s+2) Plot its pole-zero diagram and hence obtain v (t). | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Find the transmission parameters for the circuit shown below? | 5M | CO-3 | BL-3 |
|  | - Express Z parameters in terms of Y parameters. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | Determine Y-parameters using interconnection of two-port networks for the network shown in figure below | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Define constant-k filters. Derive the expression for the cut-off frequency of a constant-k low-pass filter. | 5M | CO-4 | BL-2 |
|  | - Explain the classification of filters. Describe the characteristics of low-pass, high-pass, band-pass, and band-elimination filters with suitable frequency response sketches. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | Determine the Fourier coefficients a0 , an , bn of given wave form. | 10M | CO-5 | BL-3 |

****