| Subject Code 25ES1EE105 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

NETWORK ANALYSIS

(EIE)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Differentiate Dependent and Independent sources? | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | List out the Advantages of Mesh Analysis? | 2M | CO-1 | BL-1 |
|  | State Maximum power transfer theorem? | 2M | CO-2 | BL-1 |
|  | Define RMS and Average Values? | 2M | CO-3 | BL-1 |
|  | Illustrate the equations of Transmission parameters? | 2M | CO-4 | BL-3 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Derive the expressions for Star to Delta Conversion. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | - Calculate the voltages across each resistor for given circuit below | 5M | CO-1 | BL-3 |
|  | - Find the equivalent resistance of given circuit | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Find V1 and V2 using Nodal Analysis. | 5M | CO-1 | BL-3 |
|  | - Explain the difference between super mesh and super node. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4 | - Explain the steps involved in solving Electrical networks using Mesh Analysis? | 5M | CO-1 | BL-3 |
|  | - Apply Super-mesh Analysis for the following circuit find current through each Resistor. | 5M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Derive condition for maximum power transfer theorem. | 5M | CO-2 | BL-2 |
|  | - A voltage source of 24V with internal resistance 6Ω is connected to a variable load resistance RL​. Determine: - The value of load resistance for maximum power transfer - Maximum power delivered to the load - Load current under maximum power condition | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6 | State and explain Norton’s theorem in detail with an Example? | 10M | CO-2 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Derive an Expression for Resonant Frequency for series RLC Circuit? | 5M | CO-3 | BL-2 |
|  | - Analyse the RL Circuit with AC Input and find Current equations with phasor diagrams? | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | Derive Expression for Impedance of Series RLC Circuit with Phasor Diagrams? | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Derive the Equations for converting Z-Parameters to Y-Parameters? | 5M | CO-4 | BL-2 |
|  | - Compare Low pass and High pass Filters? | 5M | C0-4 | BL-2 |
| OR |  |  |  |  |
| 10 | Derive Transmission parameters of two port network? | 10M | CO-4 | BL-3 |

****