| Subject Code 25PC1EI101 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

SENSORS AND TRANSDUCERS

(EIE)

Time: 3 hours                                                                          		 	               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10

|  | Differentiate between an active transducer and passive transducer. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define Piezo-resistive effect | 2M | CO-2 | BL-1 |
|  | Explain the working principle of LVDT | 2M | CO-3 | BL-1 |
|  | What are the applications of thermocouples? | 2M | CO-4 | BL-2 |
|  | Explain the working principle of variable frequency sensors | 2M | CO-2 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Describe the different transduction mechanisms used in sensors and transducers with suitable examples. | 10M |  | CO-1 | BL-1 |
| OR |  |  |  |  |  |
| 2 | Discuss the classification of sensors based on principle of operation, output type, and application with suitable examples. | 10M |  | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |  |
| 3 | - Metallic resistance has a direct proportionality with temperature. Identify a transducer based on the statement with proper justification. Briefly discuss the working principle of transducer based on the statement. | 5M |  | CO-2 | BL-2 |
|  | - A thermistor is represented by RT = RA eβ/T. The change in resistance of the thermistor is 4567 Ω and 2584 Ω at temperatures 300 K and 318 K respectively. What is the value of β of the thermistor? | 5M |  | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 4 | Apply the principle of a Resistive Hygrometer to explain how humidity is measured in an industrial environment. Discuss its construction, working, characteristics, and applications with a neat diagram. | 10M |  | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | What is the necessity of differential arrangement of capacitive transducer? Explain the differential arrangement of capacitive transducer with a derivation. |  | 10M | CO-3 | BL-1 |
| OR |  |  |  |  |  |
| 6 | Why LVDT is referred as variable inductive transducer? Draw the schematic diagram of LVDT and describe its construction and working in detail | 10M |  | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Distinguish between Piezo- electric effect and Piezo-resistive effect. Also write the name of the transducers which work on these effects and corresponding physical quantities measured by these transducers. | 5M |  | CO-4 | BL-2 |
|  | - Explain the Pyroelectric Effect with the help of construction and working of a pyroelectric sensor. Mention its applications. | 5M |  | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 8 | State and explain different types of thermocouples with appropriate illustrations. Give its advantages. | 10M |  | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - Analyze the applications of Smart Sensors and explain any two. | 5M |  | CO-4 | BL-4 |
|  | - Explain the working, advantages and applications of SAW sensors. | 5M |  | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 10 | Describe how position encoders can be used. Explain their construction, working principle, types and industrial applications with suitable diagrams. |  | 10M | CO-3 | BL-3 |

****