| Subject Code 25PC1CE102 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

Surveying

(CE)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | A survey line AB has a fore bearing of 125∘30'. Calculate its back bearing. | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | List out the characteristics of Contour Map. | 2M | CO-2 | BL-1 |
|  | Define the term face left and face right in a theodolite survey. | 2M | CO-3 | BL-1 |
|  | Define the multiplying constant in tacheometric surveying. | 2M | CO-3 | BL-1 |
|  | Differentiate aerial and terrestrial photograph. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Explain the principles of surveying and classify the different types of surveying? | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2 | The following bearings were taken in running a closed compass traverse while surveying at Hyderabad. State the stations which are affected by local attraction and by how much. Determine the correct bearings. Calculate the ture bearings, if the declinations was 2 o 15’ W Line; F.B; B.B. PQ; 48o25’; 230 o 00’. QR; 177o45’; 356 o00’. RS; 104o15’; 284 o55’. ST; 165o 15; 345 o15’. TO; 259 o 30’; 79 o00’. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3 | - Explain any one method of plotting contours. | 4M | CO-2 | BL-2 |
|  | - The following offsets were taken at 10 m intervals to a boundary line: Chainage (m); 0; 30; 60; 90; 120; 150; 180; 210. Offset (m); 0; 2.50; 3.86; 4.22; 5.20; 6.50; 5.00; 7.00. Calculate the area enclosed using: a) Trapezoidal Rule, b) Simpson’s Rule | 6M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4 | - Explain the two methods of levelling. | 5M | CO-2 | BL-2 |
|  | - Discuss the effect of curvature and refraction in levelling? | 5M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5 | - Explain the method of measuring a horizontal angle by the method of repetition using a theodolite? | 5M | CO-3 | BL-2 |
|  | - A theodolite was used to observe the angle of elevation of the top of a tower as 32∘40'. The horizontal distance between the instrument station and the tower is 120 m. If the height of the instrument axis is 1.5 m, determine the height of the tower. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | - Explain the procedure for measuring vertical angles using a theodolite. | 4M | CO-3 | BL-2 |
|  | - The following observations were made on a hilltop to ascertain its elevation. The height of the target was 5m. Determine the RL of top of hill. The distance between A and B was 20m. Instrument station; Staff reading on B.M.; Vertical angle on target at hill top; RL of B.M.. A; 3.220m; 15 o 30’; 620.000m. B; 1.895m; 26 o 25’; 620.000m. | 6M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | Calculate the ordinates at 10m distance for a circular curve having a long chord of 110 m and a versed sine of 5m. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 8 | Explain in detail the procedure for tacheometric measurements with: a) inclined sight and staff held vertical, and b) inclined sight and staff held normal to the line of sight. Derive the necessary equations. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - Explain the principle of Electronic Distance Measurement (EDM) and mention its advantages in surveying? | 5M | CO-4 | BL-2 |
|  | - Explain the different segments of the Global Positioning System? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10 | Explain the working principle, components, advantages, and applications of a Total Station in modern surveying? | 10M | CO-4 | BL-2 |

****