| Subject Code 25PC1ME102 |  |  |  |  |  |  |  |  | R25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. I Year II Semester Regular Examinations, June 2026

THERMODYNAMICS

(ME)

Time: 3 hours                                                                          		 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

Steam and Data Tables are allowed.

PART–A 				             5X2=10M

|  | Define thermodynamic system and write types of systems | 2M | CO-1,4 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Write the statement of first law of thermodynamics. | 2M | CO-1,2 | BL-1 |
|  | Define refrigerator and heat pump. | 2M | CO-1,3 | BL-1 |
|  | What are the processes in Diesel cycle? | 2M | CO-2,3 | BL-1 |
|  | Define the Dalton’s Law of Partial Pressures. | 2M | CO-1,4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Explain the concept of macroscopic and microscopic view point as applied to the study of thermodynamics. | 5M | CO-1,4 | BL-1 |
|  | - Define thermodynamic process? Explain quasi-static process and its characteristic features. | 5M | CO-1,4 | BL-1 |
| OR |  |  |  |  |  |
| 2 | - Show that work is a path function, and not a property. | 5M | CO-1,4 | BL-3 |
|  | - 0.2 m3 of an ideal gas at a pressure of 2 MPa is expanded isothermally to 5 times the initial volume. It is then cooled at constant volume and then compressed back polytropically to its initial state. Determine the network done during the cycle. Take polytropic index as 1.33. | 5M | CO-1,4 | BL-4 |
| UNIT-II |  |  |  |  |  |
| 3 | - What are the limitations of first law of thermodynamics. | 5M | CO-2,3 | BL-2 |
|  | - A gas undergoes a thermodynamic cycle consisting of three processes beginning at an initial state where p1 = 1 bar, V1 = 1.5 m3 and U1 = 512 kJ. The processes are as follows: (i) Process 1–2: Compression with PV = constant. p2 = 2 bar, U2 = 690 kJ (ii) Process 2–3: W23 = 0, Q23 = –150 kJ, and (iii) Process 3–1: W31 = +50 kJ. Neglecting KE and PE changes, determine the heat interactions Q12 and Q31. | 5M | CO-2,3 | BL-4 |
| OR |  |  |  |  |  |
| 4 | - Derive S.F.E.E. stating the assumption first. | 5M | CO-2,3 | BL-3 |
|  | - A turbine operates under steady flow conditions, receiving steam at the following state: Pressure 1.2 MPa, temperature 188°C, enthalpy 2785 kJ/kg, velocity 33.3 m/s and elevation 3 m. The steam leaves the turbine at the following state: Pressure 20 kPa, enthalpy 2512 kJ/kg, velocity 100 m/s, and elevation 0 m. Heat is lost to the surroundings at the rate of 0.29 kJ/s. If the rate of steam flow through the turbine is 0.42 kg/s, what is the power output of the turbine in kW? | 5M | CO-2,3 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | - Define heat engine, heat pump and refrigerator and establish relationship between C.O.P of heat pump and refrigerator. | 5M | CO-2,3 | BL-2 |
|  | - Two reversible heat engines A and B are arranged in series, A rejecting heat directly to B. Engine A receives 200KJ at 4210C while B is in communication with a cold sink at 4.40C. If the work output of A is twice that of B. Find (i) the intermediate temperature between A and B (ii) efficiency of each engine (iii) heat rejected to cold sink. | 5M | CO-2,3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | - What do you mean by ‘Clausius inequality’? Explain | 5M | CO-2,3 | BL-2 |
|  | - Calculate the entropy change of the universe as a result of the following processes: i) A copper block of 600 g mass and with Cp of 150 J/K at 100°C is placed in a lake at 8°C. ii) The same block, at 8°C, is dropped from a height of 100 m into the lake. iii) Two such blocks, at 100°C and 0°C, are joined together. | 5M | CO-2,3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Draw the phase equilibrium diagram for a pure substance on T – S plot with relevant constant property lines and explain. | 5M | CO-2,3 | BL-2 |
|  | - A vessel of volume 0.05m3 contains a mixture of saturated water saturated stem and air temperature of 300°C. the mass the liquid present is 10kg. find the following - The pressure - The Mass of Vapor - The specific Volume - The specific Enthalpy - The specific entropy | 5M | CO-2,3 | BL-4 |
| OR |  |  |  |  |  |
| 8 | Derive an expression for thermal efficiency of Otto cycle and also draw the P-V and T-S diagrams. | 10M | CO-3,4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9 | - State and explain Dalton's law of partial pressures for a mixture of gases. | 5M | CO-1,4 | BL-2 |
|  | - Define mole fraction, volume fraction, and partial pressure in gas mixtures. Show their relationships for ideal gases. | 5M | CO-1,4 | BL-2 |
| OR |  |  |  |  |  |
| 10 | - Derive the two T-ds equations used in thermodynamics. | 5M | CO-1,4 | BL-3 |
|  | - Define the Joule–Thomson coefficient. Explain its physical significance and behaviour for ideal gases. | 5M | CO-1,4 | BL-2 |

****