| Subject Code 22PC1CE202 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December 2025

CONCRETE TECHNOLOGY

(CE)

IS 10262-2019 code book is allowed

Time: 3Hrs											      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What is Portland Pozzolana Cement (PPC)? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define coarse aggregate and fine aggregate. | 2M | CO-2 | BL-1 |
|  | What are super plasticizers? | 2M | CO-3 | BL-1 |
|  | Define gel-space ratio. | 2M | CO-3 | BL-1 |
|  | What is Nofine concrete? | 2M | CO-4 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - What is hydration of cement? Explain with graph? | 5M | CO-1 | BL-3 |
|  | - Discuss the importance of gypsum in cement manufacturing. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Explain the properties and applications of: (a) Rapid hardening cement (b) High-alumina cement (c) Sulphate resisting cement | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | - Explain sieve analysis and draw standard IS sieve sets for: Fine aggregates and write what is zone classification. | 5M | CO-2 | BL-3 |
|  | - Write the difference between Well-graded and Gap-graded aggregates. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | Define physical properties of aggregate: - i) Specific gravity ii) Water Absorption | 4M | CO-2 | BL-2 |
|  | - Classify aggregates as per shape, size and texture. | 6M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Explain segregation and bleeding. | 5M | CO-3 | BL-2 |
|  | - Explain factors influencing the workability of concrete? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Describe any two workability tests: - Slump test - Compaction factor test - Vee-Bee consistometer | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Calculate Gelspace ratio and theoretical strength of a concrete made with 500 gms of cement with a water cement ratio of 0.5 for full hydration. | 5M | CO-3 | BL-2 |
|  | - Explain modulus of elasticity of concrete and factors influencing it. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8 | What is shrinkage? Explain types of shrinkage. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the step-by-step procedure for concrete mix design as per IS 10262. | 5M | CO-4 | BL-2 |
|  | - Define fibre reinforced concrete and list different types of fibres? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | Design a concrete mix for characteristic strength of 30MPa at 28 days with a standard deviation of 5MPa. The specific gravity of FA and CA are 2.65 and 2.72 respectively. A slump of 100mm is necessary. The specific gravity of cement is 3.10, Exposure Condition: Severe. Assuming the necessary data design the mix as per IS code 10262-2019 method. | 10M | CO-4 | BL-3 |

****