| Subject Code 22PC1EC211 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

DIGITAL LOGIC DESIGN

(Common to CSE, IT, CyS, and IoT)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define 1’s complement and 2’s complement. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are basic logic gates? | 2M | CO-2 | BL-1 |
|  | Differentiate between combinational and sequential circuits. | 2M | CO-3 | BL-2 |
|  | Explain the function of a PROM. | 2M | CO-4 | BL-2 |
|  | What is a state diagram? | 2M | CO-5 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain the concept of Error Detection and Correction. | 5M | CO-1 | BL-2 |
|  | - Describe the working of Hamming Code with an example for single-bit error correction. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Perform the following conversions step by step: (a) Convert (1011101)₂ to octal and hexadecimal. (b) Convert (657)₈ to binary and then to hexadecimal. (c) Convert (587.625)₁₀ to binary. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Convert the expression F = x+(x+y)(y+z)into SSOP and SPOS. (Write min-term and max-term numbers) | 5M | CO-2 | BL-3 |
|  | - Implement the function below using NOR gates only: f=a(b+cd)+bc | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Reduce the below expression and implement using NAND gates. F(A, B, C, D) = m(1, 5, 6, 12, 13, 14) + d (2, 4) | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | Design and explain the Full Adder circuit using Half Adder. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Write short notes on: a) Binary adder–subtractor b) Priority encoder | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Describe the structure and operation of a Programmable Logic Array (PLA). | 5M | CO-4 | BL-2 |
|  | - Explain with diagram how a latch differs from a flip-flop. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | Convert D flip-flop to JK flip-flop. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | Design a Sequence Detector to detect the sequence ‘1010’ using Mealy model in non-overlapping case with T flip-flops. | 10M | CO-5 | BL-4 |
| OR |  |  |  |  |
| 10 | Design a 3-bit Synchronous Up-Counter using JK flip-flops. | 10M | CO-5 | BL-3 |

***