| Subject Code 22PC1EE201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

ELECTRICAL MACHINES - I

(EEE)

Time: 3 hours                                                                                 		    Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Write the principle of DC Generator? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define critical field resistance and critical speed? | 2M | CO-2 | BL-1 |
|  | Why DC series motor cannot be run under no-load condition? | 2M | CO-3 | BL-2 |
|  | Differentiate between the ideal and practical transformer? | 2M | CO-4 | BL-2 |
|  | Differentiate between core-type and shell-type three-phase transformers? | 2M | CO-4 | BL-2 |

PART–B			    	          	   5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the construction of DC machine with neat sketch? | 5M | CO-1 | BL-2 |
|  | - Derive the expression for force in a singly excited system. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | Differentiate between the Lap and Wave winding? also derive the EMF equation of D Generator? | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | - Explain about the voltage buildup process in DC Shunt generator? | 5M | CO-2 | BL-2 |
|  | - A shunt generator running at 500 rpm delivers 50 kW at 200 V. The armature and field resistances are 0.02 Ω and 40 Ω respectively. Calculate the (i) generated emf (ii) Power developred by aramature if brush drop of 1 V per brush. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Analyse armature reaction in DC generators? Also discuss its causes, effects, and methods of compensation? | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | - How do you conduct the Swinburne’s test. On DC shunt machine? Explain. | 5M | CO-3 | BL-2 |
|  | - A 220 V d.c series motor has armature and field resistances of 0.15 Ω and 0.10 Ω respectively. It takes a current of 30 A from the supply while running at 1000 rpm. If an external resistance of 1 Ω is inserted in series with the motor, calculate the new steady state armature current and the speed. Assume the load torque remains constant. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Distinguish various methods of speed control of DC shunt motors. | 10M | CO-4 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Discuss about transformer on no-load condition with phasor diagram? | 5M | CO-3 | BL-2 |
|  | - Derive an expression for voltage regulation of single phase transformer? | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | Explain about construction of single-phase transformers? Also derive EMF equation of transformer? | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Discuss about scott connection of a transformer? | 5M | CO-4 | BL-4 |
|  | - Explain the advantages of auto transformers. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Explain the different configurations of three-phase transformers. | 10M | CO-4 | BL-2 |

****