| Subject Code 22PC1EE202 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

ELECTROMAGNETIC FIELDS

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What does negative sign indicates in E=−∇V expression? | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is meant by dielectric polarization? | 2M | CO-1 | BL-1 |
|  | State Biot-Savart's Law and provide its mathematical expression for the magnetic field produced by a differential current element. | 2M | CO-2 | BL-2 |
|  | What is meant by Magnetic Dipole? | 2M | CO-3 | BL-1 |
|  | Differentiate between statically induced EMF and dynamically induced EMF. | 2M | CO-4 | BL-2 |

PART-B					5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Derive the expression for D due to the infinite sheet of charge placed in y=0 plane, using Gauss’s law. | 6M | CO-1 | BL-3 |
|  | - Define potential and potential difference. | 4M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 2 | A circular ring of radius ‘a’ carries a uniform charge density of ρL C/m and is in z=0 plane with axis same as z-axis. Find the electric field at any point (0,0,h). | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Short note on an isolated conductor under the influence of an applied field. | 5M | CO-1 | BL-2 |
|  | - For the current density J=10zsin2ϕaρ A/m2, find the current through the cylindrical surface ρ=2, 1≤z≤5m. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4 | What are electric boundary conditions? Derive them for the case dielectric–dielectric boundary with neat sketch. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Apply Ampere’s Circuital Law to calculate the MFI due to an infinite sheet of current. | 5M | CO-2 | BL-3 |
|  | - Find the flux crossing the portion of the plane ϕ=π/4 define by 0.01<<0.05m and 0<z<2m. A current filament of 2.5A along the z-axis is in the az direction. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6 | Determine the magnetic field intensity produced by a long straight current-carrying wire using Biot-Savart’s law. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Write about force on a differential current element placed in a magnetic field. | 5M | CO-3 | BL-2 |
|  | - What is Lorentz force equation? Explain in detail. | 5M | CO-3 | BL-1 |
| OR |  |  |  |  |
| 8 | Determine force between two straight long and parallel current carrying conductors. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Describe Maxwell’s equations for time-varying fields and explain modification from the static case. | 5M | CO-4 | BL-3 |
|  | - Derive Poynting theorem. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | - Discuss about scalar magnetic potential and its limitations. | 5M | CO-4 | BL-2 |
|  | - Derive the self-inductance of a toroid with a circular cross-section. Assume whatever necessary. | 5M | CO-4 | BL-3 |