| Subject Code 22PC1EC201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

ELECTRONIC CIRCUIT ANALYSIS

(ECE)

Time: 3 hours                                                                  			Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | List the typical values of h-parameters in CE transistor configuration. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the advantages of the Darlington amplifier? | 2M | CO-2 | BL-2 |
|  | Compare current-series and current-shunt feedback amplifiers. | 2M | CO-3 | BL-2 |
|  | Explain the condition for generating oscillations. | 2M | CO-4 | BL-1 |
|  | Compare BJT and FET amplifiers. | 2M | CO-5 | BL-2 |

PART-B		     		         5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain the effect of coupling and bypass capacitors on frequency response of CE amplifier. | 5M | CO-1 | BL-2 |
|  | - Analyse CC amplifier using approximate h-parameter model and derive the expression for its Current gain. | 5M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2 | Analyse CE amplifier using exact h-parameter model and derive the expression for its Current gain, Input resistance, voltage gain, and Output resistance. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | - Draw the Hybrid-π model of a transistor in Common Emitter configuration. Also mention typical hybrid-pi parameter values. | 5M | CO-2 | BL-2 |
|  | - Derive the short circuit current gain for CE amplifier. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Illustrate different types of coupling schemes used in multi-stage amplifiers with necessary diagrams. Also discuss the advantages and disadvantages of each type. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Discuss the effects of the methods of sampling and mixing upon the output resistance Rof and the input resistance Rif with feedback. | 5M | CO-3 | BL-1 |
|  | - A Voltage-Shunt feedback amplifier employs a basic amplifier with input and output resistance of 1kΩ each and gain A = 2000. If feedback factor β=0.1, find (i) closed loop gain, (ii) Rif and (iii) Rof of closed loop amplifier. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | Analyse Voltage-Series feedback amplifier with necessary diagram using BJT and derive the expression for closed loop gain, Input resistance, and output resistance. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain the working principle of RC phase shift oscillators. | 7M | CO-4 | BL-2 |
|  | - In a Colpitt’s oscillator, C1 = 0.2 μF and C2 = 0.02 μF. If the frequency of the oscillator is 10 KHz, find the value of the inductor. | 3M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | Draw the circuit diagram of Hartley oscillator. Also derive the expression for its frequency of oscillation. | 10M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | Draw and analyze the common source amplifier using the JFET and compare with MOSFET amplifier. | 10M | CO-5 | BL-4 |
| OR |  |  |  |  |
| 10 | Draw the circuit diagram and small signal equivalent circuit of MOSFET Common Source amplifier. Also derive the expression for its voltage gain for different loads. | 10M | CO-5 | BL-2 |

****