| Subject Code 22PC1EI201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December 2025

ELECTRONIC DEVICES AND CIRCUITS-I

(EIE)

Time: 3 hours                                                                                        Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | List the applications of Zener diode. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the need of biasing? | 2M | CO-2 | BL-2 |
|  | Draw BJT transistor small signal low frequency hybrid model | 2M | CO-3 | BL-1 |
|  | What is the function of bypass capacitor | 2M | CO-3 | BL-2 |
|  | Define positive and negative feedback of the amplifier. | 2M | CO-3 | BL-1 |

PART-A					5X2=10M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the operation of a PN junction diode in forward biased and reverse biased condition | 5M | CO-1 | BL-2 |
|  | - Derive the expression for Diffusion capacitance CD. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 2. | - Derive the expression for ripple factor of a Full-Wave center-tap rectifier with capacitor filter. | 4M | CO-2 | BL-3 |
|  | - Voltage of 10sinwt is applied to Full Wave Rectifier with load resistance of 2KΩ. Calculate Maximum DC Voltage, RMS, Ripple Factor, PIV and Efficiency. | 6M | CO-4 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | - Explain the common base input and output characteristics with necessary graphs. | 5M | CO-2 | BL-2 |
|  | - A transistor operating in CB configuration has Ic = 2.98mA, IE = 3mA and IC0 =0.01 mA. What current will flow in the collector circuit of this transistor when connected in CB configuration with base current of 30µA. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | - What is meant by Q-point? Explain how to select Q-point for an amplifier. | 4M | CO-3 | BL-3 |
|  | - Determine the dc bias voltage VCE and current IC for the voltage-divider configuration if VCC=18V, R1=39KΩ, R2= 3.9KΩ, RC=4KΩ, RE=1.5KΩ and β=140. | 6M | CO-3 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | - With the help of characteristic curves, explain the principle of operation of ‘n’ channel Junction Field Effect Transistor. Mark its regions of operation on the curve. | 5M | CO-1 | BL-2 |
|  | - Explain the working of ‘n’ channel depletion MOSFET. Draw the drain and transfer characteristics, explain them in detail. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6. | - Derive the expressions for input impedance, current gain, voltage gain and output impedance of Common Emitter amplifier using exact h-parameter model | 5M | CO-3 | BL-2 |
|  | - In a single stage CE amplifier with Rs=1kΩ, R1=50kΩ, R2=2kΩ, Rc=1kΩ, RL=1.2kΩ, hfe=50, hie=1.1kΩ, hoe=25μA/V and hre=2.5х10-4 . Calculate current gain, input impedance, voltage gain and output impedance | 5M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | Derive the expression for CE short circuit current gain Ai as a function of frequency using hybrid π model. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8. | Design a single state CE transistor amplifier with resistive load and analyze it with hybrid π model. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | What are the different types of negative feedback? Briefly explain how the input and output impedances of an amplifier are affected by the different types of negative feedback. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | - State and explain Barkhausen’s criteria. | 5M | CO-4 | BL-1 |
|  | - Explain the working of Hartley oscillator. Also derive the expression for its frequency of oscillations. | 5M | CO-4 | BL-3 |

***