| Subject Code 22PC1CE203 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December 2025

FLUID MECHANICS

(CE)

Time: 3 hours                                                                                 		       Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define Kinematic and Dynamic Viscosity. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Stream function and velocity potential function. | 2M | CO-2 | BL-1 |
|  | Differentiate between Notch and Weir. | 2M | CO-2 | BL-2 |
|  | Differentiate between Total Energy line and Hydraulic Grade Line. | 2M | CO-2 | BL-2 |
|  | Define Magnus effect. | 2M | CO-1 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | A square plate of size 1 m x 1 m and weighing 350 N slides down in inclined plane with a uniform velocity of 1.5m/s. The inclined plane is laid on a slope of 5 vertical to 12 horizontal and has an oil film of 1mm thickness. Calculate the dynamic viscosity of oil. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 2 | - Derive an expression for the depth of centre of pressure from free surface of liquid of an inclined plane surface submerged in the liquid | 5M | CO-2 | BL-2 |
|  | - Find the total pressure and position of centre of pressure on a triangular plate of base 2.0 m and height 3.0 m which is immersed in water in such a way that the plan of the plate makes an angle of 600 with the free surface of the water. The base of the plate is parallel to water surface and is at a depth of 2.5 m from water surface | 5M | CO-3 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Differentiate between laminar flow and turbulent flows, and rotational and irrational flows. | 5M | CO-1 | BL-2 |
|  | - Determine whether the following velocity components satisfy the continuity equation. u = cx, v = -cy u = -cx/y, v = c log xy | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 4 | - What are the forces acting on fluids? State the Bernoulli’s equation and discuss the significance of different terms. | 5M | CO-1 | BL-2 |
|  | - A pipe line tapers from 1.5m in diameter at higher end to 1.0 m diameter at lower end in 400m length at a slope of 1 in 100. The pressure at the higher end is 75 KPa. If the discharge is 60 m3 / minute, find the pressure at lower end. Neglect losses. | 5M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | a) Explain the working of a venturimeter with a neat sketch | 5M | CO-2 | BL-2 |
|  | b) An venturimeter with throat diameter 15cm is inserted in a pipe of 30cm diameter. The pressure gauges fitted to the inlet and throat of the venturimeter given readings of 14.715N/cm2 and 9.81N/cm2. Find the rate of flow of water through the pipe. Take Cd=0.98. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 6 | The head of water over an orifice of diameter 100mm is 10 m,. The water coming out from orifice is collected in a circular tank of diameter 1.5m. The rise of water level in this tank is 1.0 m in 25 seconds. Also the co-ordinates of a point on the jet, measured from vena-contracta are 4.3 m horizontal and 0.5 m vertical. Find the co-efficients, Cd, Cv and CC. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - List out the minor losses in closed conduit flow and discuss their significance | 5M | CO-2 | BL-2 |
|  | - A 6 cm diameter pipe has a discharge of 450 1iter/min. At a section the pipe has a sudden expansion to a size of 9 cm diameter. If the pressure just upstream of the expansion is 20 kN/m2, calculate the pressure just after the expansion. Assume the pipe to be horizontal. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 8 | Determine the difference in the elevations between the water surfaces in the two tanks which are connected by a horizontal pipe of diameter 300 mm and length 400m. The rate of flow of water through the pipe is 300 liters/s. Consider all losses and take the value coefficient of friction f=.008 | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - What is a boundary layer ? Differentiate between a laminar and turbulent boundary layer. | 5M | CO-1 | BL-2 |
|  | - Explain the characteristics of laminar and turbulent boundary layer along long thin plate | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 10 | For the velocity profile uU=2yδ-yδ2determine the displacement thickness, momentum thickness and energy thickness is the laminar boundary layer. | 10M | CO-4 | BL-4 |

****