| Subject Code 22ES1EE201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

FUNDAMENTALS OF ELECTRICAL ENGINEERING

(EIE)

Time: 3 hours                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | What are the losses in a DC motor? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define voltage regulation of a transformer? | 2M | CO-2 | BL-1 |
|  | What is the main advantage of using the Laplace transform in transient analysis? | 2M | CO-3 | BL-1 |
|  | What is the relationship between Z and h parameters? | 2M | CO-4 | BL-1 |
|  | Define cut-off frequency? What is the characteristic equation. | 2M | CO-4 | BL-1 |

PART-B					5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Derive the EMF equation of a DC generator | 5M | CO-1 | BL-2 |
|  | - A 4 pole DC generator is delivering 20 A to a load of 10 Ω. If the armature resistance is 0.5 Ω and the shunt field resistance is 50 Ω, calculate the induced EMF of the machine. Allow a drop of 1V per brush. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | - Describe with suitable diagrams the principle of operation of DC generator? | 5M | CO-1 | BL-3 |
|  | - A 4-pole DC motor is fed at 440V and takes an armature current of 50A. The resistance of armature circuit is 0.28Ω. The armature winding is wave connected with 888 conductors and useful flux per pole is 0.023 wb. Calculate the speed of the motor. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | - Derive the EMF equation of a transformer | 5M | CO-2 | BL-2 |
|  | - Draw the Phasor diagram of ideal transformer on load at leading and lagging power factor. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | - Discuss the torque slip characteristics of a 3-φ induction motor. | 5M | CO-2 | BL-2 |
|  | b) The no. of turns on the primary and secondary windings of a single phase transformer are 350 and 35 respectively. If the primary is connected to a 2.2 KV 50 Hz supply, determine the secondary voltage. | 5M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - The circuit shown in fig consists of resistance, inductance and capacitance in series with a 100 V constant source when the switch is closed at t = 0. Find the current transient? | 5M | CO-3 | BL-3 |
|  | - Derive the transient response of a series RL circuit with DC input. Sketch the variation of current and of the voltage across the inductor? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | How can we classify that the given circuit is of first order or second order? Define time constant of RL and RC networks and explain the significance of time constant? | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Determine the admittance parameters for the ᴨ network shown in fig. and draw the Y parameter equivalent circuit? | 5M | CO-4 | BL-2 |
|  | - ABCD parameters are also known as transmission parameters and are derived from basic two port network parameters, show that for reciprocal linear time invariant two port network AD –BC =1 | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | Find ABCD parameters of the network shown in fig. Also find the image parameters for the network. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - What are the classifications of filters? Discuss them briefly. | 5M | CO-4 | BL-2 |
|  | - Design a constant K low pass T and ᴨ section filter to be terminated in 600Ω having cut-off frequency of 3 KHz? | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10 | Design a m- derived low pass filter having cut off frequency of 1 KHz design impedance of 400Ω and the resonant frequency of 1100 Hz? | 10M | CO-4 | BL-4 |

****