| Subject Code 22PC1AE205 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December 2025

FUNDAMENTALS OF THERMODYNAMICS

(AE)

Time: 3Hrs										      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define a thermodynamic system. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is steady flow process? | 2M | CO-1 | BL-1 |
|  | State the second law of thermodynamics | 2M | CO-1 | BL-2 |
|  | Draw the P-V diagram for a pure substance | 2M | CO-1 | BL-2 |
|  | What are the causes of irreversibility? | 2M | CO-2 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Explain briefly zeroth law of thermodynamics. | 5M | CO-1 | BL-2 |
|  | - Compare open and closed thermodynamic sysytem | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | To a closed system 150 kJ of work is supplied. If the initial volume is 0.6 m3 and pressure of the system changes as p = 8–4V, where p is in bar and V is in m3, determine the final volume and pressure of the system. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - In an internal combustion engine, during the compression stroke the heat rejected to the cooling water is 50 kJ/kg and the work input is 100 kJ/kg. Calculate the change in internal energy of the working fluid stating whether it is a gain or loss. | 5M | CO-2 | BL-2 |
|  | - Explain the First Law of Thermodynamics as referred to closed systems undergoing a cyclic change. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 4 | In a gas turbine unit, the gases flow through the turbine is 15 kg/s and the power developed by the turbine is 12000 kW. The enthalpies of gases at the inlet and outlet are 1260 kJ/kg and 400 kJ/kg respectively, and the velocity of gases at the inlet and outlet are 50 m/s and 110 m/s respectively. Calculate : (i) The rate at which heat is rejected to the turbine, and (ii) The area of the inlet pipe given that the specific volume of the gases at the inlet is 0.45 m3/kg. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Draw the P–V and T–S diagrams of a Carnot cycle and explain the four processes. | 5M | CO-3 | BL-2 |
|  | - A heat engine receives heat at the rate of 1500 kJ/min and gives an output of 8.2 kW. Determine : (i) The thermal efficiency ; (ii) The rate of heat rejection. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | 0.04 m3 of nitrogen contained in a cylinder behind a piston is initially at 1.05 bar and 15°C. The gas is compressed isothermally and reversibly until the pressure is 4.8 bar. Calculate : (i) The change of entropy, (ii) The heat flow, and (iii) The work done. Sketch the process on a p-v and T-s diagram. Assume nitrogen to act as a perfect gas. Molecular weight of nitrogen = 28. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - Briefly explain Dalton’s law of partial pressures | 5M | CO-2 | BL-2 |
|  | - What amount of heat would be required to produce 4.4 kg of steam at a pressure of 6 bar and temperature of 250°C from water at 30°C ? Take specific heat for super-heated steam as 2.2 kJ/kg K. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 8 | A vessel having a capacity of 0.05 m3 contains a mixture of saturated water and saturated steam at a temperature of 245°C. The mass of the liquid present is 10 kg. Find the following : (i) The pressure, (ii) The mass, (iii) The specific volume, (iv) The specific enthalpy, (v) The specific entropy, and (vi) The specific internal energy. | 10M | CO-2 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the concept of available and unavailable energy. When does the system become dead? | 5M | CO-2 | BL-2 |
|  | - Explain the various processes of a otto cycle with P-V and T-S diagrams. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 10 | In a steam power cycle, the steam supply is at 15 bar and dry and saturated. The condenser pressure is 0.4 bar. Calculate the Carnot and Rankine efficiencies of the cycle. Neglect pump work. | 10M | CO-3 | BL-4 |

****