| Subject Code 22PC1AE204 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December 2025

MECHANICS OF MATERIALS

(AE)

Time: 3 hours                                                                                 		              Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A				         5X2=10M

|  | Define the term toughness and represent toughness material in under the area of its stress-strain diagram? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Write the relationship between rate of loading, shear force and bending moment | 2M | CO-1 | BL-2 |
|  | Clearly state the assumptions made while develop the theory of simple being. | 2M | CO-3 | BL-2 |
|  | What is meant by maximum and minimum principal stress? | 2M | CO-2 | BL-1 |
|  | According to theory of buckling distinguish long and short columns. | 2M | CO-3 | BL-1 |

PART-B				       5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Two parallel walls 6m apart are stayed together by a steel rod 25 mm diameter passing through metal plates and nuts at each end. The nuts are tightened, when the rod is at a temperature of 1000C. Determine the stress in the rod, when the temperature falls down to 600C and the ends yield by 1 mm? Take Es=205 GPa and s=11x10-6 /oc. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2 | Sketch the stress-strain diagram of mild steel and explain the salient points of the curve. | 10M | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |  |
| 3 | A solid cylindrical shaft is to transmit 300 kW power at 100 r.p.m. If the shear stress is not to exceed 80N/ mm2, find its diameter. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 4 | Determine load P such that reactions at supports A and B are equal in the beam shown in Fig. 1. Also draw shear force and bending moment diagrams marking the values at salient points. Fig.1 | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | Two beams are simply supported over the same span and have the same flexural strength. Compare the weights of these two beams, if one of them is solid and the other is hollow circular with internal diameter half of the external diameter? | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | A beam has cross-section as shown in Fig. 2. If the shear force acting on this is 25 kN, draw the shear stress distribution diagram across the depth. Fig. 2 | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | - A thin cylindrical pressure vessel of diameter 600 mm and thickness 12 mm carries an internal pressure of 2.5 MPa. Calculate the Maximum stress. | 5M | CO-2 | BL-2 |
|  | - Derive the equations for principal stresses and maximum shear stress for a two-dimensional stress system. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 8 | The stress element as shown in Fig. 3. Determine the principle stresses and the maximum shear stresses. Also find (i) The magnitude of resultant stress on principal plane. (ii) The directions of the principal planes and maximum shear planes. Fig. 3 | 10M | CO-2 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - Derive an equation for crippling load for the column with both ends fixed. | 5M | CO-3 | BL-4 |
|  | - What is meant by an equivalent length of column? How to calculate equivalent length of columns for different end conditions? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 10 | Determine the equation of the deflection curve for a cantilever beam loaded by a uniformly distributed load w per unit length, and a concentrated force P at the free end using double integration method. Also determine the maximum deflection. | 10M | CO-3 | BL-3 |

****