| Subject Code 22PC1ME201 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

MECHANICS OF SOLIDS

(ME)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define normal stress and normal strain, and explain their significance in axial loading. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Differentiate flexural and torsional rigidity. | 2M | CO-2 | BL-2 |
|  | What do you mean by pure and non-uniform bending? | 2M | CO-3 | BL-2 |
|  | Draw Mohr’s circle and mention its features. | 2M | CO-4 | BL-2 |
|  | What are the boundary conditions for determining deflections in simply supported beam? | 2M | CO-5 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Draw the stress-strain curve for low carbon steel and explain the salient points. | 5M | CO-1 | BL-2 |
|  | - A stepped circular steel bar subjected to various forces as shown in Fig. If E = 200 kN/mm2, determine the change in its length. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | A steel tube of 30 mm external diameter and 20 mm internal diameter encloses a copper rod of 15 mm diameter to which it is rigidly joined at each end. If, at a temperature of 100C there is no longitudinal stress, calculate the stresses in the rod and tube when the temperature is raised to 2000C. Take E for steel and copper as 2 × 105 N/mm2 and 1 × 105 N/mm2 respectively. The value of co-efficient of linear expansion for steel and copper is given as 11 × 10-6 per oC and 18 × 10-6 per 0C respectively. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Relate the load, shearforce and bending moment with necessary expressions. State the importance of shearforce and bending moment diagrams for beams. | 5M | CO-2 | BL-2 |
|  | - Determine support reactions and shear force at sections E, F and G of beam as shown in the figure below. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Two shafts of the same material and of same lengths are subjected to the same torque, if the first shaft is of a solid circular section and the second shaft is of hollow circular section, whose internal diameter is 2/3 of the outside diameter and the maximum shear stress developed in each shaft is the same, compare the weights of the shafts. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5.. | - Explain the theory of simple bending and state assumptions. | 5M | CO-3 | BL-2 |
|  | - A timber beam of rectangular section is to support a load of 20 kN uniformly distributed over a span of 3 m when beam is simply supported. If the depth of section is to be twice the breadth, and the bending stress in the timber is not to exceed 7 N/mm2, find the dimensions of the cross-section. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | - Derive the expression for maximum shear stress in the beam τ=VAyIb | 5M | CO-3 | BL-2 |
|  | - A timber beam of rectangular section is simply supported at the ends and carries a point load at the centre of the beam. The maximum bending stress is 12 N/mm2 and maximum shearing stress is 1N/mm2, find the ratio of the span to the depth. | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Derive the expressions normal and shear stresses transformed in a structure subjected to biaxial loading. | 5M | CO-4 | BL-3 |
|  | - An element in biaxial stress is subjected to stresses σx=-50 MPa and σy=20 MPa. Using Mohr’s circle, determine the stresses acting on an element oriented at a counterclockwise angle θ=25°from the x axis. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | A cylindrical shell is 3 m long, and is having 1 m internal diameter and 15 mm thickness. Calculate the maximum intensity of shear stress induced and also the changes in the dimensions of the shell, if it is subjected to an internal fluid pressure of 1.5 N/mm2. | 10M | CO-5 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Derive the differential equation of elastic curve. | 5M | CO-5 | BL-3 |
|  | - A beam 6 m long, simply supported at its ends, is carrying a point load of 50 kN at its centre. The moment of inertia of the beam (i.e. I) is given as equal to 78 × 106 mm4. If E for the material of the beam = 2.1 × 105 N/mm2, calculate deflection at the centre of the beam. | 5M | CO-5 | BL-4 |
| OR |  |  |  |  |
| 10. | A beam of length 6 m is simply supported at ends and carries two-point loads 48 kN and 40 kN at a distance of 1m and 3m respectively from the left support. Find (i) Deflection under each load (ii) Maximum deflection, and (iii) the point at which maximum deflection occurs. Given E= 2.1 x 105 N/mm2 and I=85 x 106 mm4. | 10M | CO- | BL-4 |

****