| Subject Code 22PC1EC203 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

SIGNALS AND SYSTEMS

(Common to ECE and EIE)

Time: 3 hours                                                                                 		    	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define continuous time unit impulse and unit step functions. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | State Parseval’s theorem. | 2M | CO-2 | BL-1 |
|  | Find the inverse Laplace transform of 1s+a2 | 2M | CO-3 | BL-2 |
|  | Define the bandwidth of a signal and a system. | 2M | CO-4 | BL-1 |
|  | State and prove the Time reversal property of z-transform. | 2M | CO-5 | BL-2 |

PART–B			    	          	   5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | a) Find whether the following signal is periodic or not? If periodic, determine the fundamental period: x(t)=3sin(200πt) + 4cos⁡(100t). | 5M | CO-1 | BL-2 |
|  | b) A rectangular function f(t) is defined by f(𝑡) = 1 ; 0 <𝑡< 𝜋, and 𝑓(𝑡) = −1; 𝜋< 𝑡<2𝜋. Approximate this function by a waveform sin(t) over the interval 0, 2π, by finding the optimum coefficient value such that mean square error is minimum. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | a) If a signal x(t) is defined as given in below Figure, find and sketch the following signals: i) x(2t-4) ii) x(-2t+4) iii) x (t/2) | 5M | CO-1 | BL-2 |
|  | b) Sketch the following signals: i) x(t)= u(t)-2u(t-1)+u(t-2) ii) x(t)= u(t+2)-u(t-2) iii) x(t)= r(t)-r(t-1)-r(t-3)+r(t-4) where u(t) and r(t) represent unit step function and unit ramp function respectively. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | - Find the Fourier transform of a rectangular pulse. Sketch the signal and its Fourier spectrum. | 5M | CO-2 | BL-2 |
|  | b) Find the Trigonometric Fourier series expansion of halfwave rectified sine wave shown below. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | - State and prove differentiation and integration properties of Fourier transform. | 5M | CO-2 | BL-3 |
|  | - Consider the periodic square wave x(t) as shown in below Figure. Determine the complex Exponential Fourier series of x(t). | 5M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Determine the Laplace transform of the following: i) x(t) =cos3(3t) ii) x(t) = t sin(at). | 5M | CO-3 | BL-2 |
|  | - Explain the difference between the following systems: - Linear and non-linear systems. - Time variant and time invariant systems. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Find the Transfer function and impulse response of the following circuit: | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Find the auto correlation of x(t)= A sin(ω0t). | 5M | CO-4 | BL-2 |
|  | - Discuss Causality and Paley-Wiener criterion for physical realization of a system. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | State and prove properties of auto correlation function. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | The signal x(t) = 10 cos (10πt) is sampled at a rate of 8 samples per second. Plot the spectrum for ¦¦≤ 30π. Can the original signal be recovered from samples? Discuss. | 10M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10 | Explain and prove the properties of ROC of Z-transform. | 10M | CO-5 | BL-2 |

****