| Subject Code 22PC1CE201 |  |  | HT No. |  |  |  |  |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular/Supplementary Examinations, December, 2025

STRENGTH OF MATERIALS-I

(CE)

Time: 3Hrs											      Max.Marks:60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define the terms elasticity and plasticity. | 2M | CO1 | BL1 |
| --- | --- | --- | --- | --- |
|  | What is a composite bar? | 2M | CO1 | BL2 |
|  | What is the difference between simply supported beam and a cantilever beam? | 2M | CO2 | BL2 |
|  | List the assumptions made in the theory of simple bending. | 2M | CO3 | BL1 |
|  | Define bending stress and shear stress. | 2M | CO4 | BL1 |

PART–B			    	          	   5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - A circular rod of diameter 20 mm and 500 mm long is subjected to a tensile force of 45 kN. The modulus of elasticity for the material is 2.1x105 N/mm2. Find the stress, strain and the elongation of circular rod. | 6M | CO-1 | BL-2 |
|  | - Draw the stress strain diagram for mild steel and explain the salient points. | 4M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | - Define the following terms (i) Hooke’s law & factor of safety and (ii) Lateral & longitudinal strains. | 4M | CO-1 | BL-1 |
|  | - A steel bar 300 mm long, 50 mm wide and 40 mm thick is subjected to a pull of 300 kN in the direction of its length. Determine the change in volume. Take E=2x105N/mm2 and μ=0.25. | 6M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | A steel rod of 3 cm diameter is enclosed centrally in a hollow copper tube of external diameter 5 cm and internal diameter of 4 cm. The composite bar is then subjected to an axial pull of 45000 N. If the length of each bar is equal to 15 cm, determine: (i) The stresses in the rod and tube, and (ii) The load carried by each bar. | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4 | A steel rod of 3 cm diameter and 5 m long is connected to two grips and the rod is maintained at a temperature of 95oC. Determine the stress and pull exerted when the temperature falls to 30oC, if (i) The ends do not yield, and (ii) The ends yield by 0.12 cm. Take E=2x105N/mm2 and α=12x10-6/oC. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | A cantilever beam of length 3 m carries a uniformly distributed load of 2.5 kN/m length over the whole length and a point load of 3.5 kN at the free end. Draw SFD and BMD for the cantilever beam. Also, determine the maximum values of SF and BM. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 6 | A simply supported beam of length 8 m carries point loads of 4 kN and 7 kN at distances 3 m and 6 m from the left end. Determine the maximum bending moment and shear force. Also, plot the variation of SF and BM for the beam. | 10M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | A timber beam of rectangular section is to support a load of 30 kN uniformly distributed over a span of 4 m when the beam is simply supported. If the depth of section is to be twice the breadth, and the stress in the timber is not to exceed 8 N/mm2, find the dimensions of the cross section. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8 | - Define section modulus and explain the terms involved. | 2M | CO-3 | BL-1 |
|  | - A timber beam 120 mm wide and 200 mm deep is simply supported over a span of 4 m. The beam carries a UDL of 2.8 kN/m over the entire length. Find the maximum bending stress induced. | 8M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | A steel beam of I–section, 200 mm deep and 160 mm wide has 16 mm thick flanges and 10 mm thick web. The beam is subjected to a shear force of 200 kN. Determine the shear stress distribution over the beam cross section and plot its variation. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10 | The cross section of a joist is a T-section of 120 mm x 80 mm x 12 mm with 120 mm side horizontal. Find the maximum intensity of the sear stress and sketch the distribution of shear stress across the section if it has to resist a shear force of 75 kN. | 10M | CO-4 | BL-4 |

****