| Subject Code 22PC1EE205 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

CONTROL SYSTEMS

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					     5X2=10M

|  | What are the advantages of a feed back in control system. | 2M | CO-2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Type 1 and Order 2 of a system. | 2M | CO-2 CO-3 | BL-2 |
|  | Define Phase Margin? | 2M | CO-3 | BL-1 |
|  | Draw electrical circuit for lag compensator. | 2M | CO-4 | BL-1 |
|  | Write the three properties of State Transition Matrix. | 2M | CO-2 | BL-1 |

PART–B					    5X10=50M

| UNIT-I |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - Derive the transfer functionC(S)R(S) for the system as shown in given figure using block diagram reduction techniques. | 5M | CO-2 |  | BL-2 |  |
|  | - Explain about open loop and closed loop system with examples | 5M | CO-1 |  | BL-2 |  |
| OR |  |  |  |  |  |  |  |
| 2 | - Give the F-V, F-C Analogy for following block diagram | 5M | CO-2 |  | BL-2 |  |
|  | - State different rules which govern the block diagram reduction technique to obtain transfer function of a system. | 5M | CO-1 |  | BL-2 |  |
| UNIT-II |  |  |  |  |  |  |  |
| 3 | - Derive the time response of the second order system when excited with unit step input for critically damped case. | 7M | CO-2 |  | BL-3 |  |
|  | b) Derive the time domain specifications of the second order system for (i) rise time (tr) (ii) peak time (tp) (iii) Maximum overshoot (Mp) | 3M | CO-1 |  | BL-3 |  |
| OR |  |  |  |  |  |  |  |
| 4 | - Using R-H Criteria, determine the locations of the roots of the following characteristic equations and comment on stability. | 5M | CO-3 |  | BL-2 |  |
|  | - A unity feedback system has an open loop transfer function Determine K so that the system will have a damping ratio 0.5. For this value of K, determine peak over shoot and time for peak over shoot for the unit step input. | 5M | CO-3 |  | BL-3 |  |
| UNIT-III |  |  |  |  |  |  |  |
| 5 | Sketch the root locus for the following unity feedback open loop transfer function | 10M | CO-3 |  | BL-4 |  |
| OR |  |  |  |  |  |  |  |
| 6 | Draw the Bode plot for the transfer function From the graph determine i) Gain Crossover Frequency ii) Phase Crossover Frequency iii) G.M and P.M iv) Comment on stability | 10M | CO-3 |  | BL-3 |  |
| UNIT-IV |  |  |  |  |  |  |  |
| 7 | - Realize lead compensator with the help of basic electrical components. | 5M | CO-1 CO-4 |  | BL-3 |  |
|  | - How does the steady state value of the system changes when the following controllers are added to the system. i) PI Controller ii) PID Controller | 5M | CO-1 CO-4 |  | BL-2 |  |
| OR |  |  |  |  |  |  |  |
| 8 | Consider a unity feedback system with open loop transfer function, . Design a Lag compensator so that the phase margin of the system is 300at a frequency of 1.2 rad/sec. | 10M |  | CO-4 |  | BL-3 |
| UNIT-V |  |  |  |  |  |  |  |
| 9 | - Obtain the transfer function of the system defined by the following state space equations x1(t)x2(t)x3(t)=-2100-31-3-4-5 x1(t)x2(t)x3(t)+001 u(t) yt=010 x1(t)x2(t)x3(t) | 5M | CO-3 |  | BL-4 |  |
|  | - Determine to state transition matrix for the following system, where A=20-1-1; B=11; x0=10 | 5M | CO-3 |  | BL-4 |  |
| OR |  |  |  |  |  |  |  |
| 10 | Write the state equation for the system shown in figure in which x1, x2 and x3 constitute the state vector. Determine whether the system is completely controllable. | 10M | CO-2 CO-3 |  | BL-3 |  |

****