| Subject Code 22PC1AE303 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

DESIGN FOR AUTOMOTIVE COMPONENTS

(AE)

Time: 3 hours                                                                                 	             Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					     5X2=10M

|  | State maximum shear stress theory of failure? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is notch sensitivity in materials? | 2M | CO-2 | BL-1 |
|  | Define the throat of a fillet weld. | 2M | CO-3 | BL-1 |
|  | Write the applications of helical spring. | 2M | CO-4 | BL-1 |
|  | What is the difference between rigid and flexible couplings? | 2M | CO-4 | BL-1 |

PART–B				              5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) Explain the basic procedure involved in design of any machine component. | 5M | CO-1 | BL-2 |
|  | b) A hollow shaft of 40 mm outer diameter and 25 mm inner diameter is subjected to a twisting moment of 120 N-m, simultaneously, it is subjected to an axial thrust of 10 kN and a bending moment of 80 N-m. Calculate the maximum compressive and shear stresses. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2. | The load on a bolt consists of an axial pull of 10 kN together with a transverse shear force of 5 kN. Find the diameter of bolt required according to 1. Maximum principal stress theory; 2. Maximum shear stress theory; 3. Maximum principal strain theory; 4. Maximum strain energy theory; and 5. Maximum distortion energy theory. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3. | a) Illustrate how the stress concentration in a component can be reduced. | 5M | CO-2 | BL-2 |
|  | b) Write Soderberg's equation and compare it with Gerber and Goodman equations. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4. | A machine component is subjected to a flexural stress which fluctuates between + 300 MN/m2 and – 150 MN/m2. Determine the value of minimum ultimate strength according to 1. Gerber relation; 2. Modified Goodman relation; and 3. Soderberg relation. Take yield strength = 0.55 Ultimate strength; Endurance strength = 0.5 Ultimate strength; and factor of safety = 2. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |  |
| 5. | a) Explain the advantages of bolts over welded joints. | 5M | CO-3 | BL-2 |
|  | b) Pressure of steam acting on the head of the cylinder is 0.7 MPa, when the diameter of the cylinder is 300 mm. The load is held in position by 12 bolts of size M20 × 2.5. The soft copper gasket is used to make the joint. Determine the stresses induced in bolt. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6. | Explain the design procedure of a welded joint when a load acts eccentrically in the plane of welds. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7. | Design a close coiled helical compression spring for a service load ranging from 2250 N to 2750 N. The axial deflection of the spring for the load range is 6 mm. Assume a spring index of 5. The permissible shear stress intensity is 420 MPa and modulus of rigidity, G = 84 kN/mm2. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 8. | What are the advantages and disadvantages of flat belts over V-belts. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9. | A solid circular shaft is subjected to a bending moment of 3000 N-m and a torque of 10 000 N-m. The shaft is made of 45 C 8 steel having ultimate tensile stress of 700 MPa and a ultimate shear stress of 500 MPa. Assuming a factor of safety as 6, determine the diameter of the shaft. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 10 | Describe rigid shaft couplings mentioning the application of each type. | 10M | CO-4 | BL-2 |

****