| Subject Code 22PC1CE301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

DESIGN OF REINFORCED CONCRETE STRUCTURES

(CE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer any ONE question in PART-A.

Answer any THREE questions from PART-B.

All questions carry equal marks in PART-B.

Note: IS 456 Code book, SP 16 Design charts are permitted to use:  Assume any missing data suitably.

PART-A

Answer any ONE question.								 		1X24 = 24 M

| 1. | Design a RCC slab for a room measuring 4mX5m inside dimensions. The slab carries a live load of 2kN/m2 and is finished with 20mm thick granolithic finishing having unit weight of 24kN/m3. Use M20 and Fe415 materials and the slab is simply supported on all four edges with corners free to lift. Width of supporting wall is 300mm. Deflection and shear checks are not necessary. Sketch the reinforcement detailing. | CO-3 | BL-4 |
| --- | --- | --- | --- |
| 2. | Design a spread footing for a column, 400 mm × 400 mm, carrying an axial (service) load of 3000 kN. Assume an allowable soil pressure of 350 kN/m2 at a depth of 1.5 m below ground. Assume M 30 concrete and Fe 500 steel. | CO-4 | BL-4 |

PART-B

Answer any THREE questions.									3X12 = 36 M

| 3. | A rectangular beam of span 7 m with clear distance resting on two walls of 300 mm wide simple supports, is to carry a live load of 20 kN/m. Using M20 and Fe 415 steel, design the beam section. | CO-1 | BL-4 |
| --- | --- | --- | --- |
| 4. | Explain the following section. - Balanced Section - Under-Reinforced Section - Over-Reinforced Section | CO-1 | BL-2 |
| 5. | The effective flange width of a T-beam is 1200mm with depth of flange as 120mm. The overall depth of the section is 800mm with width of the web as 200mm. The beam is reinforced with 4-20mm dia Fe500 steel. Determine the ultimate moment carrying capacity of the beam, if concrete used is M25 grade. Assume moderate environment exposure conditions | CO-1 | BL-3 |
| 6. | Design the reinforcement in a short square column of size 600 mm x600 mm subjected to an axial load of 2000 kN under service dead load and live load. The column has an unsupported length of 3.6 m and effectively held in position and restrained against rotation in both ends. Use M 25 concrete and Fe 500 steel. | CO-2 | BL-4 |
| 7. | Explain different types of footings and the process of selecting each type. Explain with neat sketches. | CO-4 | BL-2 |

***