| Subject Code 22PC1EC302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

DIGITAL SIGNAL PROCESSING

(Common to EIE and ECE)

Time: 3 hours                                                                 		Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Explain the block diagram representation of LCCD equations. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Define Discrete Fourier Transform. | 2M | CO-2 | BL-1 |
|  | What are Butterworth and Chebyshev approximations? | 2M | CO-3 | BL-2 |
|  | Compare FIR and IIR filters. | 2M | CO-4 | BL-1 |
|  | List applications of Multirate Signal Processing. | 2M | CO-5 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Explain the concept of frequency domain representation of discrete-time signals. | 5M | CO-1 | BL-2 |
|  | - Analyse DFS representation of periodic sequences. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |  |
| 2 | A discrete-time periodic signal x[n] has period N = 4, with values x0=1, x1=2, x2=3,x3=4 is given. Find its Discrete Fourier Series (DFS) coefficients. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |  |
| 3 | - Describe the steps involved in using the DFT to perform circular convolution and explain how it can be extended to compute linear convolution. | 5M | CO-2 | BL-2 |
|  | - Compute the 4-point DFT of the sequence xn={1, -1, 1, -1} using the DFT formula: Xk= n=0N-1xne-j2πknN, k=0, 1, 2, 3 | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4 | Compare and contrast the Radix-2 Decimation-in-Time (DIT) and Decimation-in-Frequency (DIF) FFT algorithms in terms of computations. Explain the differences in their processing stages. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5 | - Explain design procedure for low pass digital Butterworth IIR filter. | 5M | CO-3 | BL-2 |
|  | - Compare Butterworth and Chebyshev filters based on their frequency responses. | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |  |
| 6 | An analog low-pass Butterworth filter is given by: Hs= 1s2+ 3s+2 and the bilinear transformation: s= 2T . 1- z-11+ z-1, Using the bilinear transformation method with sampling period T = 1 sec, find the digital filter equation transfer function H(z). | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Define FIR filter and its characteristics. | 5M | CO-4 | BL-2 |
|  | - Explain Rectangular and Hamming windows. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 8 | Design a linear phase FIR band-pass filter using the Hamming window method. The desired passband is between ω1=0.3π and ω2=0.7π. Use a filter length of N=9.Compute and write down the impulse responseh[n]. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9 | - Define down sampling, up sampling, and sampling rate conversion. | 5M | CO-5 | BL-2 |
|  | - Explain the process of decimation and interpolation with examples | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |  |
| 10 | Analyze the different types of finite word length effects in digital filters, including overflow, round-off noise, and limit cycles | 10M | CO-5 | BL-4 |

***