| Subject Code 22PC1CE303 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

ESTIMATION AND COSTING

(CE)

Time: 3 hours                                                                                 	      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					     5X2=10M

|  | Define detailed estimate and rough estimate. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | How is the quantity of woodwork for doors and windows measured? | 2M | CO-2 | BL-2 |
|  | What is rate analysis? List its components. | 2M | CO-2 | BL-2 |
|  | Differentiate between open tender and limited tender. | 2M | CO-4 | BL-2 |
|  | What is the purpose of a sinking fund? | 2M | CO-3 | BL-2 |

PART–B					    5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Summarize the general rules of Measurement & also explain the deductions and additions to be applied in the case of estimation of plastering. | 5M | CO-1 | BL-2 |
|  | b) Prepare a preliminary estimate for a framed two storied office building having a carpet area of 400 Sqm for each floor. Assume areas occupied by corridor, verandah, lavatories, staircase etc. as 25% of built-up area and that occupied by walls and columns as 8.5% of the same. The following details may be used for estimation. - Built-up area rate for Ground Floor (Excluding Foundation) = Rs.1,500 per Sqm - Built-up area rate for First Floor = Rs.1,650 per Sqm - Extra for foundation = 20% of Superstructure cost - Extra for Special Architectural treatment = 1% of Building cost - Extra for Water supply and sanitary = 7% of Building cost - Extra for Electrical Installation = 8% of Building cost - Extra for Work charge establishment = 10% of Overall cost - Extra for Other works = 5% of Building cost | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | Prepare a detailed estimate of the following R.C.C works of a R.C.C framed building from the given section, elevation and plan (Fig.1.A, 1.B). i) columns, ii) Roof beam iii) Sand filling in plinth iv) Earthwork in filling. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - i) Calculate the area of the sides of portion of a bank for a length of 400 meter. The heights of banks at the two ends being 2.50m & 3.50m and the ratios of the side slopes 2:1. ii) If the side slopes are to be provided with 15cm thick stone pitching, calculate the cost of pitching at the rate of 550 per cum. | 5M | CO-2 | BL-2 |
|  | b) Explain in detail the methods of estimation of roads. | 5M | CO-2 | BL- |
| OR |  |  |  |  |
| 4. | Estimate the cost of earthwork for a portion of a road from the following data. Road width at the formation surface is 8m. Side slopes 2:1 in banking and 1.5:1 in cutting. Length of chain is 30m. | 10M | CO-2 | BL-3 |
|  | Chainage; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30. GL; 71.2; 71.25; 70.9; 71.25; 70.8; 70.45; 70.2; 70.35; 69.1; 69.7; 68.9. FL; 70; Upward gradient of 1 in 200. |
| UNIT-III |  |  |  |  |
| 5. | a) Discuss the factors affecting rate analysis. | 5M | CO-3 | BL-2 |
|  | b) Calculate the quantity of materials for first class brick work in cement mortar(1:4) in super structures, ground for 1m3 of work. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | A RCC beam has the following data: Length = 6 m, cross section is 300 mm wide and 600 mm deep, tensile reinforcement of 5 Nos. 20 mm diameter bars, compression reinforcement of 3 Nos. 12 mm diameter bars, 2 legged 8 mm diameter vertical stirrups at 250 mm spacing. Estimate the RCC work. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Explain Earnest Money Deposit and Security Deposit. | 5M | CO-4 | BL-2 |
|  | - Explain the procedure for the preparation and submission of tender papers. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | a) Discuss various types of contracts and their advantages. | 5M | CO-4 | BL-2 |
|  | b) Explain the rights and responsibilities of parties to a contract. | 5M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Define the following terms: (i) Net income, (ii) Scrap value (iii) Annuity (iv) Capital cost | 4M | CO-3 | BL-2 |
|  | - Explain the depreciation method of valuation. | 6M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 10. | In a plot of land costing Rs. 2,50,000/-, a building is constructed at a total cost of Rs. 20,00,000/- inclusive of sanitary, water supply units, electrical installations etc. The building has four flats intended for four prospective tenants. Returns of 8% on the cost of construction and 5% on the cost of land are expected. Life of the building is expected to be 50 years. Sinking fund need to be created at 5% interest. Annual repairs are likely to be at 1.5% of cost of construction. Other expenses including taxes are likely to be 25% of the net returns. Calculate the standard rent for each flat. | 10M | CO-3 | BL-3 |

***

Fig. 1.A

Fig. 1.B