| Subject Code 22PE1EC320 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

FUNDAMENTALS OF SIGNALS AND SYSTEMS

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Explain the concept of orthogonality in functions with an example | 2M | CO1 | L2 |
| --- | --- | --- | --- | --- |
|  | Find the Fourier transform of unit impulse signal | 2M | CO2 | L1 |
|  | Define impulse response and explain how it is used in LTI system analysis | 2M | CO3 | L2 |
|  | State the conditions for distortion less transmission through a LTI system | 2M | CO4 | L1 |
|  | Explain how Z-Transforms are useful in discrete time system analysis | 2M | CO3 | L3 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Determine whether the following discrete time signals are periodic or not? If periodic determine the fundamental period? i) xn=cos0.001πn ii) xn=cosn8cosnπ8 | 5M | CO-1 | BL-2 |
|  | - Find and sketch the even and odd components of the following xn=e-n4un xt=t, 0≤t≤12-t, 1≤t≤2 | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2 | Explain how a square wave with “amplitude A and time period T” can be approximated using sinusoidal functions and derive the expression for mean square error | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |  |
| 3 | - Find the Fourier Transform of i) sgnt ii) Rectangular Pulse | 7M | CO-2 | BL-2 |
|  | - Derive the relationship between Trigonometric Fourier series and exponential Fourier Series | 3M | CO-2 | BL-3 |
| OR |  |  |  |  |  |
| 4 | Sketch the signal xt=t2 for all t and find the trigonometric Fourier series over the interval (-1,1). | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5 | - Find the following system is i) Linear or Nonlinear ii) time invariant or time variant yn=xn+x(n-1) | 5M | CO-3 | BL-2 |
|  | - Define LTI system and write the properties of the Linear time invariant system. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |  |
| 6 | - Apply convolution to find the convolution when xt=e-atUt, ht=U(t) | 5M | CO-3 | BL-3 |
|  | - Find the Laplace transform of xt=e-tUt-e-3tU-t | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Explain the relation between convolution and correlation. | 5M | CO-4 | BL-2 |
|  | - The auto correlation function of a non-periodic signal is given by Rτ=e-t22σ2 Determine the Power spectral density and normalized average power | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |  |
| 8 | Find the inverse Laplace Transform of Fs=5(s+2)(s-1) if the ROC is i)-2<Res<1 ii) Re(s)>1 | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | - State and prove Sampling theorem | 5M | CO-5 | BL-2 |
|  | - Define Nyquist interval, apply sampling theorem to find Nyquist sampling interval of the following signal. xt=cos100πt+sin120πt | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |  |
| 10 | The system is described by the difference equation. yn-32yn-1+14yn-2=xn-5x(n-1) Find the transfer function and impulse response of the system. | 10M | CO-5 | BL-2 |

***