| Subject Code 22PC1EI301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

INDUSTRIAL PROCESS CONTROL INSTRUMENTATION

(EIE)

Time: 3 hours                                                                                Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A				           5X2=10M

|  | Differentiate between a P–I converter and an I–P converter. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | State the significance of mass balance principle in mathematical modeling. | 2M | CO-1 | BL-1 |
|  | Draw a circuit diagram of an electronic PI controller. | 2M | CO-2 | BL-2 |
|  | Define feed-forward control. How is it combined with feedback control to reduce disturbances? | 2M | CO-3 | BL-2 |
|  | A control valve has a flow coefficient Cv=12. Find the flow rate Q (in gpm) of fluid with specific gravity of 1000 kg/m3 for a pressure drop of ΔP=4. | 2M | CO-4 | BL-2 |

PART–B				         5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Illustrate the construction and working principle of a piezo-electric pressure transducer. Mention any three advantages of using piezo-electric materials in dynamic pressure measurement. | 5M | CO-1 | BL-2 |
|  | - A differential pressure transmitter (4 to 20 mA) is used to measure pressure difference in a line. If the output is 12 mA at a calibrated span of 0–100 kPa, determine the pressure being measured. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | - Explain the principle of Venturi meter based on Bernoulli’s theorem. Draw a labelled diagram and list any two errors affecting Venturi meter measurement. | 5M | CO-1 | BL-2 |
|  | - A Venturi meter has an upstream diameter of 80 mm and throat diameter of 40 mm. If the differential pressure reading is 4 KPa, calculate the flow rate (m³/s). Take discharge coefficient Cd = 0.98 and density of water ρ = 1000 kg/m³. | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Summarize the objectives of process control and explain the terms process variable, manipulated variable, and disturbance with examples. | 5M | CO-2 | BL-2 |
|  | - Compare servo and regulatory control problems. Provide one industry example for each. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | - Find the mathematical model of a single-tank liquid level system assuming linear resistance to flow. Start from the mass balance equation. | 5M | CO-1 | BL-2 |
|  | - A tank of cross-sectional area A = 1.5 m² receives an inflow of 0.06 m³/s, with an outflow of 0.02 m³/s at a certain instant. Compute the rate of change of level (dh/dt). | 5M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Compare ON–OFF and Proportional (P) controller modes. Discuss their characteristics and mention one industrial application for each. | 5M | CO-2 | BL-2 |
|  | - Describe the characteristics of a PI controller and explain how integral action eliminates steady-state error. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6 | - Explain the Process Reaction Curve (PRC) method for tuning a PI controller. | 5M | CO-3 | BL-2 |
|  | b) Explain the working of IMC-PID controller. | 5M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain the working principle of cascade control with neat sketches. Discuss how inner and outer loops improve disturbance rejection. | 5M | CO-3 | BL-2 |
|  | b) Illustrate the working of split range controller. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | a) With a block diagram, explain the concept of feed-forward control combined with feedback. State two conditions required for perfect feed-forward compensation. | 5M | CO-3 | BL-3 |
|  | b) A mixing process requires a constant ratio of Fuel: Air = 1:5. If the air flow-rate measured is 200 kg/hr, compute the required fuel flow signal and draw a simple ratio control scheme. | 5M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | a) Explain the construction and working of pneumatic actuators used in control systems. Discuss their advantages and limitations. | 5M | CO-4 | BL-2 |
|  | ### b) Derive the expression for the force developed by a hydraulic actuator given: Supply pressure = 2 MPa, Piston diameter = 50 mm , Calculate the output force. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | a) With neat sketches, compare the construction, working, and applications of globe valve and butterfly valve as final control elements. | 5M | CO-4 | BL-2 |
|  | ### b) Draw and explain the ideal control valve characteristics for Linear, Equal percentage and Quick opening type valves. | 5M | CO-4 | BL-2 |

***