| Subject Code 22PE1EC321 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

INTRODUCTION TO VLSI DESIGN

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | State Moore’s law? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define Figure of merit of MOS Transistor. | 2M | CO-2 | BL-1 |
|  | Explain stick diagram and layout diagram? | 2M | CO-3 | BL-2 |
|  | Define sheet resistance and what is the use of it? | 2M | CO-4 | BL-1 |
|  | Define the difference between FinFET and MOSFET transistor? | 2M | CO-4 | BL-4 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Define SSI, MSI, LSI and VLSI? | 5M | CO-1 | BL-1 |
|  | b) Describe the operation of Depletion mode NMOS Transistor. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Discuss the fabrication process of CMOS using N-Well and Twin-Tub method with neat sketches? | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | Derive an expression for Drain current of CMOS transistor in different regions of operation? | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 4. | Derive an expression for pull-up to pull-down ratio for an NMOS inverter driven by another NMOS inverter through pass transistor? | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | a) With neat flowchart explain VLSI design flow? | 5M | CO-3 | BL-3 |
|  | b) Draw and Explain the Schematic for CMOS inverter with truth table. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | a) Define scaling? Explain constant voltage scaling with any three effects of scaling on device characteristics? | 5M | CO-3 | BL-2 |
|  | b) Draw the schematic, stick diagram, layout of NAND gate. Use color coding. | 5M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | a) Explain with waveforms propagation delay, risetime and falltime delays of CMOS inverter? | 6M | CO-4 | BL-3 |
|  | b) Discuss in detail the characteristics of CMOS Transmission gates. | 4M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | - Design a Two input NOR gate using Pass transistor logic? | 5M | CO-4 | BL-4 |
|  | b) Discuss how layer choices can affect the yield and reliability of ICs. | 5M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | a) Design a one bit parity generator using only XOR gates and draw the Schematic of it. | 5M | CO-5 | BL-4 |
|  | (b) Implement a 4-to-1 multiplexer in CMOS logic (show schematic). | 5M | CO-5 | BL-4 |
| OR |  |  |  |  |
| 10. | Discuss briefly about a) FinFET b) GNRFET | 5M 5M | CO-5 CO-5 | BL-2 BL-2 |

***