| Subject Code 22PC1ME303 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

MECHANICAL ENGINEERING DESIGN

(ME)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

(Note : Permit design data books)

PART–A					     5X2=10M

|  | What are preferred Numbers? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Draw S-N curve and label salient features. | 2M | CO-1 | BL-1 |
|  | Compare temporary joint and permanent joint | 2M | CO-2 | BL-2 |
|  | Mention various flat belt drives. | 2M | CO-4 | BL-1 |
|  | Classify the rolling contact bearings. | 2M | CO-3 | BL-1 |

PART–B					     5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Discuss the general design procedure in machine design. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | A cylindrical shaft made of steel of yield strength 700 MPa is subjected to static loads consisting of bending moment 10 kN-m and a torsional moment 30 kN-m. Determine the diameter of the shaft using (a) Maximum principle stress theory (b) Maximum shear stress theory. Assume factor of safety of 2. Take E = 210 GPa and poisson's ratio = 0.25. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Briefly discuss the equivalent twisting moment and equivalent bending moment. | 5M | CO-1 | BL-2 |
|  | - Describe the factors affecting the endurance strength of a component. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 4. | A circular cross section cantilever beam having length 130 mm. subjected to a cyclic transverse load of varying form -150 N to 350 N, FOS is 2, theoretical stress concentration factor is 1.4, ultimate strength is 540 MPa, yield strength is 320 MPa. Size correction factor is 0.85. Endurance limit is 275 MPa and surface correction factor is 0.9. Determine the diameter of the beam by Goodman method. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | For supporting the travelling crane in a workshop, the brackets are fixed on steel columns as shown in Fig. The maximum load that comes on the bracket is 12 kN acting vertically at a distance of 400 mm from the face of the column. The vertical face of the bracket is secured to a column by four bolts, in two rows (two in each row) at a distance of 50 mm from the lower edge of the bracket. Determine the size of the bolts if the permissible value of the tensile stress for the bolt material is 84 MPa. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6. | A double – riveted double – strap butt joint is used to connect two plates, each of 12 mm thickness, by means of 16 mm diameter rivets having a pitch of 48 mm. The rivets and plates are made of steel. The permissible stresses in tension, shear, and compression are 80, 60, and 120 N/mm2 respectively. Find the efficiency of the joint. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | A-V belt is to transmit 20 kW from a 250 mm pitch diameter sheave to a 900 mm diameter pulley. The centre distance between the two shafts is 1000 mm. The groove angle is 40° and the coefficient of friction for the belt and sheave is 0.2 and the coefficient of friction between the belt and flat pulley is 0.2. The cross-section of the belt is 40 mm wide at the top, 20 mm wide at the bottom and 25 mm deep. The density of the belt is 1000 kg / m3 and the allowable tension per belt is 1000 N. Find the number of belts required | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8. | A spur gear drive is required to transmit a maximum power of 22.5 kW. The velocity ratio is 1:2 and r.p.m. of the pinion is 200. The approximate centre distance between the shafts may be taken as 600 mm. The teeth has 20° stub involute profiles. The static stress for the gear material (which is cast iron) may be taken as 60 MPa and face width as 10 times the module. Find the module, face width and number of teeth on each gear. Check the design for dynamic and wear loads. The deformation or dynamic factor in the Buckingham equation may be taken as 80 and the material combination factor for the wear as 1.4. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | Design a journal bearing for a centrifugal pump running at 1440 r.p.m. The diameter of the journal is 100 mm and load on each bearing is 20 kN. The factor ZN/p may be taken as 28 for centrifugal pump bearings. The bearing is running at 75°C temperature and the atmosphere temperature is 30°C. The energy dissipation coefficient is 875 W/m2/°C. Take diametral clearance as 0.1 mm. | 10M | CO3 | BL-4 |
| OR |  |  |  |  |
| 10. | Discuss the design procedure of Piston for an internal combustion engine. | 10M | CO-3 | BL-2 |

****