| Subject Code 22PC1EC301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

MICROPROCESSORS AND MICROCONTROLLERS

(ECE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A				     5X2=10M

|  | Draw and discuss the TCON format in 8051 Microcontroller. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | How many interrupt sources are available in 8051 and what are they? | 2M | CO-1 | BL-1 |
|  | Give details of all the LCD Pin descriptions and the command codes. | 2M | CO-3 | BL-2 |
|  | What is the function of privileged and unprivileged access? | 2M | CO-1 | BL-2 |
|  | Write the sign extension instructions of Cortex-M3with an example? | 2M | CO-2 | BL-1 |

PART–B				    5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the 8051-microcontroller architecture and their functions with neat diagrams? | 5M | CO-1 | BL-2 |
|  | - Explain the memory organization of 8051 microcontroller? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | - Discuss interrupt vector, priority and internal interrupt design using 8051 microcontroller. | 5M | CO-2 | BL-2 |
|  | - Explain the addressing modes in 8051 microcontroller with examples | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Explain the steps to configure Timer 0 in Mode 1 (16-bit Timer) for generating a delay. Provide the calculation formula for determining the 2kHz square wave with program? (Assume that XTAL=11.0592 MHz) | 5M | CO-3 | BL-3 |
|  | - Write a program to send the string ”Microcontrollers” through the serial port of the 8051 microcontrollers. Assume a baud rate of 2400. (Assume that XTAL=11.0592 MHz) | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 4. | - Write an assembly program that reads data from P1 and writes to P2 continuously while giving a copy to the serial COM port transferred serially. Assume XTAL=11.0592 MHz, with baud rate 4800. | 5M | CO-3 | BL-4 |
|  | - Develop a program to create a square wave of 80% duty cycle on bit 7 of port 1. Use registers R0, R1, and R2 for delay. | 5M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Explain how to interface a relay with the 8051 microcontrollers to control an AC load along with program. | 5M | CO-4 | BL-4 |
|  | - How is an LCD interfaced with the 8051? Write a program to display “Hello” on a first line and “World” on second line of 16x2 LCD using the 8051. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 6. | - Discuss the Nested Vectored Interrupt Controller architecture. | 5M | CO-1 | BL-2 |
|  | - Draw and explain the memory map of ARM Cortex-M3 processor. | 5M | CO-1 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | - Explain the concept of unaligned data transfers in Cortex-M3. What are its advantages and restrictions compared to aligned access | 5M | CO-1 | BL-2 |
|  | - Explain the bit-band operation in Cortex-M3. How does it enhance bit manipulation efficiency? | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 8. | - Explain the issues of pipelining in Cortex-M3 Processor? | 5M | CO-1 | BL-2 |
|  | - Identify the memory access attributes of Cortex-M3 Processor? | 5M | CO-1 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the ARM Cortex-M3 instruction set architecture in detail? | 5M | CO-1 | BL-2 |
|  | - Explain with examples how branch and link and branch exchange instructions are used in subroutine and exception handling. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 10 | Demonstrate the organization and architecture of CMSIS and specify the benefits of CMSIS. | 10M | CO-1 | BL-3 |

****