| Subject Code 22PE1CE301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular /Supplementary Examinations, December, 2025

PAVEMENT ANALYSIS AND DESIGN

(CE)
(Use of Design Aids is allowed)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define Equivalent Single Wheel Load (ESWL) | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the variables considered in pavement design. | 2M | CO-1 | BL-2 |
|  | Explain the concept of radius of relative stiffness in rigid pavement analysis. | 2M | CO-2 | BL-2 |
|  | What are the advantages of Mechanistic–Empirical design over empirical design? | 2M | CO-3 | BL-2 |
|  | Define the term overlay with a neat figure. | 2M | CO-4 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain in detail the factors affecting pavement design such as wheel load, subgrade support, and environmental conditions | 5M | CO-1 | BL-2 |
|  | - Explain the process how axle load survey is carried in the field for the determination of vehicle damage factor | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | - A highway carries 4000 commercial vehicles per day in both directions. Determine the design traffic in million standard axles (msa) for a design life of 15 years. Assume growth rate = 7.5%, VDF = 3.5, and lateral distribution = 0.75, 2-lane divided carriageway | 5M | CO-1 | BL-3 |
|  | - Explain the different types of pavements based on structural composition | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Explain temperature stress in rigid pavement analysis using Westergaard’s concepts with figures. | 5M | CO-2 | BL-2 |
|  | - Determine the deflection on a two-layer pavement having surface modulus E1 = 120 MPa, subgrade modulus E2 = 65 MPa, and thickness = 500 mm when loaded with a pressure of 560 kPa over a 150 mm radius. Assume any other missing data suitably. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Determine the Stresses at interior, edge and corner regions of a cement concrete using Westergaard’s stress equation using following data. Wheel load=5100kg, a=15cm, E=3x105kg/cm2, h=20cm, Poisson ratio=0.15, K=7.5 kg/cm3, b=14.0 cm | 10M | CO-2 | BL-3 |
| s UNIT-III |  |  |  |  |
| 5. | - Explain the design steps of flexible pavement using IRC:37–2018 method. | 5M | CO-3 | BL-2 |
|  | - Explain the design of flexible pavement based on AASHTO method | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | Design a bituminous layer thickness as per IRC: 37-2018 for flexible pavement with the following data (i) 4-lane divided traffic (ii) initial traffic in the year of completion of construction is 5000 cvpd, (iii) traffic growth rate 6%, (iv) design period 20 years, (v) VDF 5.2, (vi) effective CBR of soil sub-grade 7%, (vii) Marshall parameters Va = 3% , Vbe = 11.5, (viii) Resilient Modulus of HMA layer 3000MPa, (ix) Computed horizontal and vertical strain from IITPAVE 0.000146 and 0.000243 | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Discuss the steps in the design of rigid pavement using IRC:58–2015 guidelines. | 5M | CO-3 | BL-2 |
|  | - A cement concrete pavement has a thickness of 18cm and lane width of 3.5m. Design the tie bars along the longitudinal joints using the data given below. Allowable working stress in steel tie bars, SS=1400kg/cm2 Unit weight of CC, W=2400kg/m3 Maximum value of coefficient of friction=1.2 Allowable bond stress in deformed bars, Sb=24.6kg/cm2 | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8. | Design and detail dowel bars at expansion joints of a concrete pavement of thickness 20cm, and design wheel load of 4000 kg. Assume load capacity of dowel system as 40% of design wheel load. Joint width 25mm, permissible stress in dowel bars, shear stress 1000 kg/cm2 and flexural stress 1400 kg/cm2. Bearing stress in concrete permissible is 100 kg/cm2, Radius of relative stiffness (l) =80cm | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | - Explain the procedure for overlay design over flexible pavements using Benkelman Beam Deflection data. | 5M | CO-4 | BL-3 |
|  | - Write a short note on white topping and its applications. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | Benkelman beam deflection studies were carried out on 15 selected points on a stretch of flexible pavement during summer season using a dual wheel load of 4085kg, and tyre pressure of 5.6kg/cm2. The deflection values obtained in mm after making necessity corrections are given below. Design traffic is 1800 cvpd, determine the thickness of bituminous overlay required, if the pavement temperature during test was 300C and moisture content correction factor is 1.3. Assume traffic growth rate as 7.5%. Assume any other missing suitably 1.40, 1.32, 1.25, 1.35, 1.48, 1.60, 1.65, 1.55, 1.45, 1.40, 1.36, 1.46, 1.50, 1.52, 1.45. | 10M | CO-4 | BL-4 |

****