| Subject Code 22PC1EE301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

POWER ELECTRONICS

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					     5X2=10M

|  | Draw the static and dynamic characteristics of an SCR indicating the details | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Compare three-pulse and six-pulse converters based on ripple content and output voltage, | 2M | CO-2 | BL-2 |
|  | How inductor and capacitor values are chosen in the design of a buck converter. | 2M | CO-3 | BL-2 |
|  | Draw the output voltage and current waveforms of a single-phase bridge inverter for RL load. | 2M | CO-3 | BL-2 |
|  | Write the principle of Cyclo-converters. | 2M | CO-2 | BL-1 |

PART–B					    5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - List and explain the important specifications and voltage ratings of SCRs used in power circuits. | 5M | CO-1 | BL-2 |
|  | - Draw the neat structure of power MOSFET. Explain its operation with the help of output and transfer characteristics. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | - What is the need for Gate Driver circuit? Draw and explain Gate Driver circuit of power IGBT. | 5M | CO-2 | BL-2 |
|  | - Write short notes on various GaN/SiC power devices and their applications in converters. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Describe the effect of source inductance on the performance of a single-phase full converter and derive the modified output voltage equation. | 5M | CO-2 | BL-2 |
|  | - A 230 V, 50 Hz supply feeds a single-phase full converter with RL load. - Z = 5 Ω, firing angle α = 60°. - Find the average output voltage and current. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | - Derive the average output voltage of a three-phase full converter RL load in terms of supply voltage and firing angle. A 400 V, 3-phase, 50 Hz supply feeds a full converter operating at a firing angle of 45°. Find the average output voltage and determine the delay angle at which output voltage becomes zero. - Discuss the waveform of output voltage and line current in above case. | 6M 4M | CO-2 CO-3 | BL-4 BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Differentiate between isolated and non-isolated DC–DC converters with examples. - Derive the expression for the average output voltage of a buck converter in continuous conduction mode (CCM). A buck converter operates from 24 V DC supply and delivers 12 V at 2 A. Find the duty ratio and average inductor current | 3M 7M | CO-3 CO-2 | BL-2 BL-3 |
| OR |  |  |  |  |
| 6 | - Boost converter delivers 50V form 20V at 100 KHz. L=200 H and R=20. Find duty ratio and inductor peak to peak ripple current. - Derive the relationship between duty ratio (D) and output voltage (V₀) for a boost converter. Sketch the output voltage and inductor current waveforms of the boost converter during ON and OFF periods of the switch. | 4 M 6M | CO-2 CO-3 | BL-2 BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - Derive instantaneous output voltage for a single-phase full-bridge inverter. A 220 V DC input is used for a single-phase inverter using single-pulse modulation with pulse width 2d=120°. Find the RMS fundamental output voltage. - Explain the principle of Sinusoidal Pulse Width Modulation (SPWM) and define modulation index. | 6M 4M | CO-3 CO-3 | BL-3 BL-2 |
| OR |  |  |  |  |
| 8 | - Describe operation of 3 phase inverter in 1800 conduction mode. Draw line and phase voltages. - Compare Single Pulse PWM, Multi Pulse PWM in terms of harmonic content and control complexity. | 7M 3M | CO-3 CO-3 | BL-2 BL-2 |
| UNIT-V |  |  |  |  |
| 9 | With neat circuit diagram and waveforms, explain the operation of a single-phase AC voltage controller feeding a purely resistive (R) load. - Derive the expression for RMS output voltage as a function of the firing angle α. - Comment on how power delivered to the load varies with α. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 10 | - A single-phase AC voltage controller with R load is supplied by 230 V, 50 Hz AC source. If the firing angle α=60°, determine: - RMS output voltage - Load power - Power factor - What is a matrix converter? Write the advantages over conventional AC-DC-AC systems. | 6M 4M | CO-3 CO-3 | BL-3 BL-3 |

***