| Subject Code 22PC1EE302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

POWER SYSTEMS-II

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define tree & co-tree with an example. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What are the quantities determined through load flow studies? | 2M | CO-2 | BL-1 |
|  | State the cause of short-circuit faults in a power system. | 2M | CO-3 | BL-2 |
|  | Name the three sequence networks used in unsymmetrical fault analysis. | 2M | CO-3 | BL-1 |
|  | What is the steady-state stability limit? | 2M | CO-4 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Compare the direct inspection and singular transformation methods of Ybus formation. State the advantages and limitations of each. | 5M | CO-1 | BL-2 |
|  | - Form the bus admittance matrix for the given 3-bus system where branch data given using direct inspection method. Line Bus to bus; R(pu); X(pu). 1-2; 0.05; 0.15. 1-3; 0.1; 0.3. 2-3; 0.15; 0.45. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | Using the Zbus building algorithm, form the Zbus matrix for a system whose elements are added in the following sequence: - Branch between Bus 1 and Reference Bus, impedance = j0.2 p.u. - Branch between Bus 2 and Reference Bus, impedance = j0.3 p.u. - Branch between Bus 2 and Bus 1, impedance = j0.4 p.u. Show all intermediate Zbus matrices at each step. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Write the algorithm for gauss-seidel method considering with PV Buses. | 5M | CO-2 | BL-2 |
|  | - For the two bus system with the data given below: Y11=Y22=1.6∟-800 p.u. and Y21 = Y12 = 1.9∟1000 p.u. Determine the p.u. voltage at bus 2 by the GS-method after 1-iteration. Given P2 = 0.5, Q2 = 0.3 and V1 = 1.1∟00. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Write the algorithm and draw the flow chart for Newton-Raphson method considering with PV Buses. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Define and explain the purpose of fault limiting reactors in a power system. | 5M | CO-3 | BL-2 |
|  | - Explain how short-circuit MVA is calculated for a fault occurring at different buses in a system. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | A synchronous generator and motor are rated for 30,000KVA,13.2KV and both have sub-transient reactance of 20%. The line connecting them has a reactance of 10% on the base of machine ratings. The motor is drawing 20,000KW at 0.8 pf leading. The terminal voltage of the motor is 12.8KV. When a symmetrical three-phase fault occurs at motor terminals, find the sub-transient current in generator, motor and at the fault point. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Distinguish between symmetrical and unsymmetrical faults. | 5M | CO-3 | BL-2 |
|  | - The line currents in a three-phase system are Ia=12+j24, Ib=16-j2 and Ic=-4-j6 A. Calculate symmetrical components of line currents. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | Derive the necessary equations to determine the fault current and draw the interconnection diagram for L-G fault. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Derive the swing equation for a synchronous machine connected to an infinite bus. | 5M | CO-4 | BL-2 |
|  | - Find the steady state power limit and generator internal voltage of a system consisting generator connected to infinite Bus through a series reactance of 1pu. The Generator terminal voltages is held at 1.2∟300 pu and the voltage of infinite bus is 1pu. Internal reactance of generator is 0.5 pu. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Explain the equal area criterion of transient stability for i) Sudden change in mechanical input and ii) Sudden loss of transmission line | 10M | CO-4 | BL-2 |

****