| Subject Code 22PC1ME301 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

THERMAL ENGINEERING

(ME)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					     5X2=10M

|  | Differentiate between air-standard and actual cycles. | 2M | CO-1,2 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define abnormal combustion in S.I. engines. | 2M | CO-2,3 | BL-2 |
|  | List out the methods for measurement of frictional power in IC engine. | 2M | CO-2,3 | BL-3 |
|  | List two types of energy storage systems used in electric vehicles. | 2M | CO-3,4 | BL-2 |
|  | Define relative humidity and dew point temperature. | 2M | CO-3,4 | BL-1 |

PART–B					    5X10=50M

| UNIT-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Explain the working of a four-stroke diesel engine with a neat sketch. | 10M | CO-1,3 | BL-4 |
| OR |  |  |  |  |
| 2. | Discuss the effect of heat loss, exhaust blowdown, and friction on the actual cycle efficiency. | 10M | CO-1,2 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | Explain the working of a magneto ignition system with a neat sketch. | 10M | CO-1,2 | BL-3 |
| OR |  |  |  |  |
| 4. | Describe the stages of combustion in S.I. engines and discuss the factors influencing flame speed. | 10M | CO-2,3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Explain the phenomenon of diesel knock and factors affecting delay period. | 10M | CO-2,3 | BL-2 |
| OR |  |  |  |  |
| 6. | During a test on a single-cylinder, four stroke engine, the following data was recorded: bore = 250 mm, stroke = 400 mm, speed = 250 rpm, i.m.e.p = 5.5 bar, brake load = 1.2 kN, drum dia = 1.2 m. Calculate IP, BP and mechanical efficiency. | 10M | CO-2,3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | Explain the role and types of energy storage systems in electric vehicles. | 10M | CO-1,3 | BL-3 |
| OR |  |  |  |  |
| 8. | - What are the applications of single and multi-stage compressors? - A single-stage compressor delivers 0.5 kg/min of air at 7 bar from 1 bar and 27°C. Find power required, assuming index n=1.3 and clearance 5%. | 3M 7M | CO-3,4 CO-3,4 | BL-2 BL-4 |
| UNIT-V |  |  |  |  |
| 9. | Draw and explain the vapor compression refrigeration cycle. | 10M | CO-3,4 | BL-3 |
| OR |  |  |  |  |
| 10. | Define and explain psychrometric any four processes with suitable examples. | 10M | CO-3,4 | BL-3 |

***