| Subject Code 22PC1AE302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

VEHICLE DYNAMICS

(AE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | Define lumped mass in vehicle dynamics. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is traction limit and how is it influenced by tyre-road interaction? | 2M | CO-2 | BL-1 |
|  | Mention any four aerodynamic drag reduction devices. | 2M | CO-3 | BL-2 |
|  | Distinguish between free and damped vibrations. | 2M | CO-4 | BL-4 |
|  | Define understeer gradient. | 2M | CO-5 | BL-1 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Derive the expressions for dynamic axle load under level ground conditions. b) Explain the effect of acceleration and grade on load transfer in vehicles. | 5M 5M | CO-1 CO-1 | BL-4 BL-3 |
| OR |  |  |  |  |
| 2. | A 1600 kg vehicle has a wheelbase of 2.6 m with 55% of the static weight on the front axle. Determine the dynamic axle loads when the car accelerates at 3 m/s² up a 6% grade. The CG height is 0.55 m. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | Derive the acceleration performance of a vehicle under power-limited condition. | 10M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 4. | A car of mass 1500 kg is decelerated on a level surface with constant braking effort, reducing its speed from 100 km/h to 40 km/h in 5 seconds. Calculate: (a) Deceleration (b) Stopping distance (c) Brake power (d) Energy dissipated. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | Explain aerodynamic forces acting on a car body and describe the effect of drag coefficient, lift, and side force on vehicle stability. | 10M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | Describe various aerodynamic aids and their functional significance in reducing drag and improving downforce with neat sketches. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7. | Derive the equation of motion for a single degree of freedom damped vibration system and define logarithmic decrement. | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | A quarter-car suspension system has a sprung mass of 350 kg supported by a spring of stiffness 20 kN/m and damping coefficient of 1200 Ns/m. Determine: (a) Natural frequency (b) Damping ratio (c) Amplitude ratio under a 2 Hz road excitation. | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | Explain the effect of suspension parameters on cornering performance and derive expressions for roll steer and lateral force compliance steer. | 10M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | Illustrate the relationships among characteristic speed, critical speed, and static margin for vehicle stability in high-speed cornering. | 10M | CO-5 | BL-3 |

***