| ppSubject Code 22PC1CE302 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. III Year I Semester Regular/Supplementary Examinations, December, 2025

WATER RESOURCES ENGINEERING

(CE)

Time: 3 hours                                                                         			 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART-A					5X2=10M

|  | List the forms of precipitation with examples | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Explain the Evapotrapiration. | 2M | CO-1 | BL-2 |
|  | What is Base flow in hydrograph and list types of base flow separation methods. | 2M | CO-2 | BL-1 |
|  | What is Irrigation Efficiency? List types of them. | 2M | CO-3 | BL-2 |
|  | Draw a sketch of supper passage and syphon aqueduct. | 2M | CO-4 | BL-2 |

PART-B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Explain the hydrological cycle with a neat diagram. | 5M | CO-1 | BL-2 |
|  | - Describe about Tipping bucket Rain Gauge Station with neat sketch. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2. | a) A catchment has five rain gauge stations. In a year, the annual rainfall recorded by the gauges are 78.8cm, 90.2cm, 98.6cm, 102.4cm and 70.4cm. For a 6% error in the estimation of the mean rainfall, determine the additional number of gauges needed. | 5M | CO-1 | BL-3 |
|  | b) Given 4-Rain gauges I, II, III and IV. Normal annual rainfall are 60,75,80 and 100cm respectively. The rain gauge at III went out of order during a year. If annual rainfall at other stations are 90,60 and 70cm then rainfall at station-III is? | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - List the factors affecting infiltration. Explain the working of any one type of infiltrometer. | 5M | CO-1 | BL-2 |
|  | - A reservoir has a surface area of 120 km². During a day, the Class A pan records a rainfall of 10 mm and a net rise of 3 mm in water level. If the pan coefficient is 0.75, estimate the evaporation loss from the reservoir. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4. | The mass curve of an isolated storm over a watershed is given below. Time from start (h); 0; 0.5; 1.0; 1.5; 2.0; 2.5; 3.0; 3.5; 4.0; 4.5; 5.0. Cumulative rainfall (cm); 0; 0.25; 0.27; 0.5; 1.6; 2.6; 3.5; 5.7; 6.5; 7.30; 7.70. If the storm produced a direct runoff of 3.5cm at the outlet of the watershed, estimate the average infiltration rate of the storm and duration of rainfall excess. | 10M | CO-1 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Explain the factors affecting runoff. | 5M | CO-2 | BL-2 |
|  | - The ordinates of 3-hour unit hydrograph (UH)are given below. Find ordinates of 6-hour unit hydrograph. Time; 0; 3; 6; 9; 12; 15; 18; 21; 24; 27; 30. Ordinates of 3 Hr UH; 0; 10; 25; 20; 16; 12; 9; 7; 5; 3; 0. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 6. | Compute the runoff volume due to rainfall of 15cm in a day on a 550ha watershed. The hydrological soil groups are 50% of group C and 50% of group D, randomly distributed in the watershed. The land use is 55% cultivated with good quality bunding and 45% waste land. Assume antecedent moisture condition of type-III and use standard SCS-CN equations. CNII values for land uses. Land use; Soil Group C; Soil Group D. Cultivated with good quality bunding; 76; 79. Waste land.; 85; 88. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | For the following data pertaining to a cultivated land, compute the irrigation interval and the amount of irrigation water required at each irrigation so that the moisture content at any stage does not fall below 40% of the maximum available moisture. Field capacity of soil = 30%, Permanent wilting point = 14%, Effective depth of root zone = 80 cm, Daily consumptive use = 10 mm, Density of soil = 1.3 g/cc, Application efficiency = 75%. | 10M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 8. | a) Explain the terms Duty of water and Delta of a crop. Develop relation between them. | 5M | CO-3 | BL-2 |
|  | b) Define Kor depth, Kor watering, and Kor period. Explain their importance in irrigation scheduling. | 5M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | Design an irrigation channel to carry a discharge of 6cumecs. Assume N=0.0225, m=1, bed slope = 0.25 m/km. Use Kennedy‟s theory. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10. | a) Mention the advantages of Lacey’s Theory over Kennedy’s Theory. | 5M | CO-4 | BL-2 |
|  | b) Design a regime channel for a discharge of 45 cumecs and silt factor 1.1, channel side slope of 1V:1/2H using Lacey’s theory. | 5M | CO-4 | BL-4 |

****