| Subject Code 22PC1EI402 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December 2025

ANALYTICAL INSTRUMENTATION

(EIE)

Time: 3 hours                                                                                 	  Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A				            5X2=10M

|  | A solution shows a measured resistance of 500 Ω using a conductivity cell with cell constant 1.0 cm⁻¹. Calculate the conductivity of the solution. | 2M | CO-2 | BL-3 |
| --- | --- | --- | --- | --- |
|  | State Beer–Lambert’s Law and mention the physical significance of each term in the equation. | 2M | CO-1 | BL-2 |
|  | Enlist any two technical major differences between Gas Chromatography GC) and High-Performance Liquid Chromatography (HPLC). | 2M | CO-3 | BL-1 |
|  | State the working principle of Electron Spin Resonance (ESR) Spectroscopy. | 2M | CO-1 | BL-2 |
|  | Define Derivative Thermogravimetric Analysis (DTGA) and mention how it differs from Thermogravimetric Analysis (TGA). | 2M | CO-4 | BL-2 |

PART- B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Describe the working principle, construction, and applications of a Dissolved Oxygen (DO) analyzer. | 5M | CO-3 | BL-2 |
|  | - Evaluate the effects of temperature and membrane properties on the operational stability and calibration of DO analyzers. | 5M | CO-3 | BL-4 |
| OR |  |  |  |  |
| 2. | - Explain the construction and working principle of a pH meter with a neat block diagram. | 5M | CO-3 | BL-3 |
|  | - A solution has a hydrogen ion concentration of 2.5×10−5 mol/L. calculate the pH and corresponding electrode potential (E) if the reference electrode potential is 0.00 V at 25°C. | 5M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3. | - Illustrate the principle and instrumentation of Flame Emission or Atomic Emission Spectrophotometers, mentioning major industrial uses. | 5M | CO-3 | BL-4 |
|  | - Describe the measurement of online calorific value of a fuel gas using an emission-based or thermal conductivity-type calorimeter. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 4. | - With a neat block diagram, explain the construction and working of a Double-Beam UV–Visible Spectrophotometer. | 5M | CO-3 | BL-3 |
|  | - Write short notes on the sources and detectors used in UV and Visible spectrophotometers, giving one example of each. | 5M | CO-1 | BL-2 |
| UNIT-III |  |  |  |  |
| 5. | - Analyze the principle governing chromatographic separation and classify chromatography techniques based on mobile and stationary phase characteristics. | 5M | CO-1 | BL-4 |
|  | - With a neat labeled diagram, describe the instrumentation of a Gas Chromatograph, highlighting the function of each component. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | - Discuss in detail the construction and working of packed and capillary columns used in Gas Chromatography (GC). | 5M | CO-3 | BL-2 |
|  | - A compound elutes at a retention time (tR​) of 4.5 min, while the unretained component has a time (tM​) of 1.0 min. Calculate the retention factor (k') | 5M | CO-2 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Analyze the functional principle and instrumentation layout of a Nuclear Magnetic Resonance (NMR) spectrometer with a neat block diagram. | 5M | CO-3 | BL-4 |
|  | - Describe construction and working principle of Mass spectrophotometer with neat sketch and derive expression for mass to charge ratio. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 8. | - Discuss the working principles and applications of oxygen analyzers (paramagnetic type). | 5M | CO-4 | BL-2 |
|  | - Outline the importance of pollution monitoring systems and describe the integration of NOx and CO analysers in a Continuous Emission Monitoring System (CEMS). | 5M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Explain the principle, construction, and working of a Differential Scanning Calorimeter (DSC) with a neat diagram. | 5M | CO-3 | BL-3 |
|  | - Discuss how DSC can be used to determine melting point, phase transitions, and purity of a substance. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | - Highlight the working principle and applications of a Scintillation Counter, mentioning the role of photomultiplier tubes and scintillating materials. | 5M | CO-3 | BL-3 |
|  | - Write short notes on Solid-State Detectors used for radiation detection — include types, materials, and advantages over gas-filled detectors. | 5M | CO-1 | BL-2 |

***