| Subject Code 22OE1EI401 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December 2025

DRIVE SYSTEMS FOR ROBOTS

(Open Elective)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | Differentiate between open-loop and closed-loop control. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define lead screw and ball screw. | 2M | CO-3 | BL-1 |
|  | Name any two hydraulic actuators used in robots. | 2M | CO-2 | BL-1 |
|  | Define pneumatic drive. | 2M | CO-3 | BL-1 |
|  | Define half-step and micro-step operation. | 2M | CO-4 | BL-1 |

PART–B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Illustrate the structure of a basic drive system and explain its working | 5M | CO-1 | BL-4 |
|  | - Classify various types of the drive systems and their applications in industrial robots. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | Compare and contrast open and closed loop systems with suitable examples. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Discuss the working principle and advantages of harmonic drives. | 5M | CO-2 | BL-2 |
|  | - A ball screw has a pitch of 4 mm/rev and is coupled to a stepper motor operating in half-step mode (400 steps/rev). Determine the linear movement per step of the actuator. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Analyze how drive mechanism selection affects robot speed and accuracy | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - Describe the function and operation of hydraulic pistons and transfer valves. | 5M | CO-3 | BL-2 |
|  | - Summarize the advantages and limitations of hydraulic drives in robotics. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Analyze the performance characteristics of hydraulic systems for heavy-duty applications. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - Illustrate the construction and operation of a pneumatic proportional controller. | 5M | CO-3 | BL-3 |
|  | - Discuss the advantages and applications of pneumatic systems in robotics. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8 | Explain the working of pneumatic pistons and motors with neat sketches. | 10M | CO-3 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the working principles and characteristics of DC and AC motors used in robots. | 6M | CO-4 | BL-2 |
|  | - A 4-phase stepper motor completes one revolution in 200 steps. Find the step angle and the number of steps required to rotate 90°. | 4M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Compare DC, AC, and stepper motors in terms of speed, torque, and control. | 10M | CO-4 | BL-4 |

****