| Subject Code 22PC1CE401 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December 2025

FOUNDATION ENGINEERING

(CE)

Time: 3 hours                                                                               Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A

5X2=10M

|  | Define disturbed and undisturbed soil sample. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the types of failure of finite slopes. | 2M | CO-2 | BL-1 |
|  | A retaining wall, 6 m high, retains dry sand with an angle of friction of 30° and unit weight of 16.2 kN/m3. Determine the earth pressure at rest. | 2M | CO-3 | BL-3 |
|  | Define i) Net safe bearing capacity ii) Net allowable bearing pressure. | 2M | CO-3 | BL-1 |
|  | List the types of hammers used in pile driving. | 2M | CO-4 | BL-1 |

PART- B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) Explain with a neat sketch, the electrical resistivity method of soil exploration. | 5M | CO-1 | BL-2 |
|  | b) List the salient features that should be provided in the soil exploration report. | 5M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 2. | a) Explain with neat sketch Wash boring method of soil exploration. | 6M | CO-1 | BL-2 |
|  | b) The outside and inside diameters of a sampling tube are 103mm and 98mm respectively. The cutting edge has outside and inside diameters of 108mm and 93mm respectively. Determine area ratio, inside clearance and outside clearance. | 4M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | a) Discuss the Swedish method of slices for a cohesive soil. | 5M | CO-2 | BL-2 |
|  | b) List different types of slope failures and explain with neat sketches. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | An embankment 10 m high has a slope of 1V: 2H. The soil has ϕ = 35◦, c = 10kpa, γ = 20kN/m3. A trial slip circle passing through toe has radius of 10 m with centre at the same level as top of embankment. Analyze the slope by method of slices (minimum 4 slices) to find the factor of safety. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5. | a) List the assumptions and limitations of Rankine's earth pressure theory. | 4M | CO-3 | BL-1 |
|  | b) A retaining wall, 7.5 m high, retains a cohesionless backfill. The top 3 m of the fill has a unit weight of 18 kN/m3 and ϕ = 30° and the rest has unit weight of 24 kN/m3 and ϕ = 20°. Determine the pressure distribution on the wall. | 6M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | a) Explain with neat sketches the different types of retaining walls. | 5M | CO-3 | BL-2 |
|  | b) A retaining wall, 5 m high, retains dry sand with an angle of friction of 30° and unit weight of 16.5 kN/m3. Determine the earth pressure at rest. If the water table rises to the top of the wall, determine the increase in the thrust on the wall. Assume the submerged unit weight of sand as 10 kN/m3. | 5M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | a) List the characteristics of Local shear failure. | 4M | CO-4 | BL-1 |
|  | b) Explain with neat sketch Plate load test to determine the ultimate bearing capacity of soil and the probable settlement under a given loading. | 6M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | a) Discuss the effect of ground water table on bearing capacity of soil. | 4M | CO-4 | BL-2 |
|  | b) A circular footing is resting on a stiff saturated clay with unconfined compressive strength of 250 kN/m2. The depth of foundation is 2 m. Determine the diameter of the footing if the column load is 600 kN. Assume a factor of safety as 2.5. The bulk unit weight of soil is 20 kN/m3. Terzaghi’s factors for ϕ = 0° are Nc = 5.7, Nq = 1, and Nγ = 0. | 6M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | a) Discuss the types of piles based on method of installation. | 5M | CO-4 | BL-2 |
|  | b) A group of 9 piles, 10 m long is used as a foundation for a bridge pier. The piles used are 0.3 m in diameter with centre to centre spacing of 0.9 m. The sub soil consists of clay with unconfined compressive strength of 1.5 kg/cm2. Determine the efficiency by neglecting the bearing action. Take adhesion factor as 0.9. | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 10. | a) Explain the pile load test used to determine the load carrying capacity of a pile. | 5M | CO-4 | BL-2 |
|  | b) A square group of 16 Piles of 500 mm in diameter each is arranged with a centre to centre spacing of 1 m. The piles are 12 m long and are embedded in soft clay with cohesion of 35 kN/m2. Bearing resistance may be neglected for the piles. Determine ultimate load capacity of the pile group. Take adhesion factor = 0.6. | 5M | CO-4 | BL-4 |

***