| Subject Code 22HS1EC401 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December 2025

MICROWAVE ENGINEERING

(ECE)

Time: 3 hours                                                                        	     		 	 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					     5X2=10M

|  | Derive the relation between cutoff wavelength (λc), guide wavelength (λg), and free-space wavelength (λ). | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Describe different types of matched loads used in waveguides and their importance. | 2M | CO-2 | BL-2 |
|  | Define scattering parameters (S-parameters). Why are they preferred over Z and Y parameters at microwave frequencies? | 2M | CO-3 | BL-1 |
|  | Explain the advantages and disadvantages of the magnetron? | 2M | CO-4 | BL-2 |
|  | Compare IMPATT and TRAPATT diodes? | 2M | CO-5 | BL-2 |

PART- B

5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - Derive the solution of wave equation in rectangular waveguides starting from Maxwell’s equations in TE Mode. | 6M | CO-1 | BL-3 |
|  | - Explain TE and TM modes in rectangular waveguides. How do they differ? | 4M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | Define micro strip line with a neat diagram. Explain the Quasi-TEM mode. And Derive the characteristic impedance Z0​expression for a micro strip line? | 10M | CO-1 | BL-1 |
| UNIT-II |  |  |  |  |
| 3. | - Derive the resonant frequency (f0) expression for a rectangular cavity resonator. | 5M | CO-2 | BL-3 |
|  | - A cavity resonator with dimensions a=2cm, b=1cm is excited by TE101 mode of 20GHz. Calculate the length of the cavity. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | Construct a microwave test setup using a cavity resonator, attenuator, and phase shifter. Explain flow diagram? | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - What is Faraday rotation? Explain how it occurs in ferrite circulator with neat sketches. | 7M | CO-3 | BL-2 |
|  | - Explain what a gyrator is. Show how it produces phase shift without changing magnitude? | 3M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | - Derive the S-matrix calculations of directional couplers using two hole type with neat diagrams. | 6M | CO-3 | BL-4 |
|  | - Explain about coupling factor, insertion loss, Isolation factor of a directional coupler? | 4M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Explain the limitations of conventional vacuum tubes when operating at microwave frequencies? | 3M | CO-4 | BL-2 |
|  | - Explain the working of cavity magnetron in Zero mode and Pi-mode with neat diagram. | 7M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | With a neat diagram, explain the structure and working principle of a two-cavity klystron amplifier. And Explain the velocity modulation process in a two-cavity klystron with the help of the Applegate diagram? | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9. | - Explain the Microwave power measurement using Bolometer method. | 5M | CO-5 | BL-2 |
|  | - Explain the VSWR and Describe the methods for measuring high and low VSWR’s. | 5M | CO-5 | BL-2 |
| OR |  |  |  |  |
| 10. | Explain the principle of operation of a Gunn diode using the two-valley theory. And Discuss various modes of operation of a Gunn diode and state which mode is most suitable for microwave generation. | 10M | CO-5 | BL-2 |

****