| Subject Code 22PC1EE401 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December 2025

SWITCHGEAR AND PROTECTION

(EEE)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | What are the different types of Circuit Breakers? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Write the differences between differential relay and biased differential relay. | 2M | CO-2 | BL-2 |
|  | What are the abnormal conditions may occur in a large alternator. | 2M | CO-3 | BL-2 |
|  | Explain about a Phase Comparator. | 2M | CO-4 | BL-2 |
|  | Mention the functions of a PMU. | 2M | CO-5 | BL-2 |

PART- B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | - What are the different types of air blast circuit breakers. Explain the operation of cross blast type circuit breaker. | 5M | CO-1 | BL-2 |
|  | - Describe the operation of Vacuum circuit breaker with diagram and list out its merits and demerits. | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2. | The following observations were achieved on a single frequency transient during short circuit test on a circuit breaker: Time to reach the peak restriking voltage = 40μSec, Peak restriking voltage=100kV, Calculate (i) the average RRRV and (ii) the frequency of oscillation | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3. | - Explain the following classification of relays i) Depending up on the construction and principle of operation ii) Depending upon application | 5M | CO-2 | BL-2 |
|  | - Explain the operating characteristic of mho type distance relay. | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4. | Discuss travelling wave based line directional (TW32) and Differential (TW87) Relays. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Explain Buchholz relay Protection scheme. | 5M | CO-3 | BL-1 |
|  | - Emphasize how a directional relay is used in the generator protection. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6. | Describe with the help of a neat diagram the connections of differential protection of transformer. A 3 phase 33/6.6kV star-delta connected transformer is protected by differential protection. The CTs on LV side have a ratio of 500/5A. Find the ratio of CTs on the HV side. | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7. | - Discuss how the static instantaneous over current relay works to get the directional characteristics. | 5M | CO-4 | BL-2 |
|  | - Derive the expression for general equation of a comparators. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8. | Draw and explain the circulating current type and phase splitting type amplitude comparators. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9. | - Explain the operation of a PMU with a neat block diagram. | 5M | CO-5 | BL-2 |
|  | - How Digital Relaying Algorithms are useful for analyzing power system data and detect faults. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10. | Discuss the Application of Artificial Intelligence to Power System Protection (Digital Relaying). | 10M | CO-5 | BL-3 |

***