| Subject Code 22PC1ME402 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year I Semester Regular Examinations, December 2025

TURBOMACHINERY

(ME)

Time: 3 hours                                                                                 			      Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					     5X2=10M

|  | Explain the principle of Fire and Water tube boilers. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is the condition for maximum discharge in a nozzle? | 2M | CO-2 | BL-3 |
|  | What are the methods of compounding in turbines? | 2M | CO-3 | BL-1 |
|  | What is degree of reaction in axial compressors? | 2M | CO-3 | BL-2 |
|  | Define regeneration in gas turbines. | 2M | CO-4 | BL-1 |

PART- B					5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Explain the working of bob cock and will cox boiler with neat sketch and parts. | 10M | CO-1 | BL-1 |
| OR |  |  |  |  |
| 2. | The following data were recorded from the test of surface condenser. Inlet temperature of circulating water = 21OC Exit temperature of circulating water = 35OC Vacuum in the condenser = 704.7mm of Hg Barometer reading = 760 mm of Hg Calculate the efficiency of the condenser. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3. | - Explain nozzle efficiency and derive its expression. | 5M | CO-2 | BL-3 |
|  | - Describe convergent and divergent nozzles with sketches. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4. | The steam leaves the nozzels of a single-row impulse turbine at 900 m/s. The nozzle angle is 20O and blade angles are 30O at inlet and outlet. Calculate the blade velocity and work done per kg of steam. Assume the flow over the blade is frictionless. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5. | - Explain the construction and working of an impulse turbine. | 5M | CO-3 | BL-2 |
|  | - Draw the combined velocity diagram for steam turbine. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6. | In a single-stage impulse turbine, steam at 10 bar, 300°C expands to 1 bar. Blade speed = 180 m/s, nozzle angle = 20°, blade friction coefficient = 0.9. Draw velocity diagram and find: (i) Diagram efficiency (ii) Axial thrust per kg/s (iii) Power developed(iv) Condition for maximum efficiency. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7. | - Explain centrifugal compressor working with velocity diagrams. | 5M | CO-3 | BL-2 |
|  | - Explain degree of reaction in axial compressors. | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 8. | A centrifugal compressor delivers 10 kg/s air from 1 bar, 27°C to 4 bars. Impeller tip dia = 0.6 m, speed = 15000 rpm, slip factor = 0.9, power input factor = 1.04. Find: (i) Work input (ii) Isentropic efficiency (iii) Temperature rise (iv) Impeller blade angle at exit. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9. | Explain the working of solid and liquid propellant rocket ginenes. . | 10M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10. | A gas turbine plant has pressure ratio 6, turbine inlet 1100°C, compressor inlet 27°C. Isentropic efficiencies: compressor 85%, turbine 88%. Find: (i) Thermal efficiency (ii) Work ratio (iii) Air-fuel ratio (iv) Specific work output. | 10M | CO-4 | BL-4 |

****