| Subject Code 22PE1EC414 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. IV Year II Semester Regular Examinations, May 2026

LOW POWER VLSI SYSTEMS

(ECE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Differentiate between power and energy? | 2M | CO-1 | BL-3 |
| --- | --- | --- | --- | --- |
|  | Explain Hot electron effect? | 2M | CO-2 | BL-2 |
|  | Write the disadvantages of Ripple Carry Adders. | 2M | CO-3 | BL-3 |
|  | What is binary multiplication? | 2M | CO-4 | BL-1 |
|  | Compare SRAM and DRAM? | 2M | CO-5 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Derive an expression for short circuit power dissipation with necessary equations and neat sketches. | 10M | CO-1 | BL-4 |
| OR |  |  |  |  |
| 2 | - What is switching power dissipation? How can it be minimized by reducing switching activity? | 5M | CO-1 | BL-4 |
|  | - What are different types of power dissipation in CMOS circuits and discuss any three methods in detail? | 5M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Explain drain induced barrier lowering and punch through? | 5M | CO-2 | BL-2 |
|  | - Explain the following terms with respect to low power VLSI design? i) VTCMOS ii) MTCMOS | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Compare pipelining and parallel processing approaches with suitable examples. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - Discuss power dissipation components in CMOS adders and methods to reduce power. | 5M | CO-3 | BL-2 |
|  | - Derive carry generation and propagation equations in CLA. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | Compare Ripple carry and carry save adders? | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain different multiplier architectures and compare them. | 5M | CO-4 | BL-2 |
|  | - Explain the architecture of Braun multiplier with diagram. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 8 | Explain Baugh-Wooley algorithm for signed multiplication. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the architecture and working of ROM? | 5M | CO-5 | BL-2 |
|  | - Explain 6T SRAM cell with diagram. | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10 | Explain 1-T DRAM memory cell operation and refresh mechanism in DRAM? | 10M | CO-5 | BL-2 |

****