| Subject Code 22PC1AE206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

AUTOMOTIVE ENGINES

(AE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | List the principal types of IC engines with their classification criteria. Give one application for each type. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | List the significance of the air-fuel ratio in SI engines? Differentiate between rich and lean mixtures. | 2M | CO-1 | BL-2 |
|  | State the function of a throttle position sensor (TPS) and explain how it influences fuel injection. | 2M | CO-2 | BL-1 |
|  | Define degree of supercharging. State its effect on engine power output. | 2M | CO-3 | BL-1 |
|  | Define brake thermal efficiency and mechanical efficiency. Write their mathematical expressions. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Sketch and explain the actual valve timing diagram of a four-stroke SI engine. Identify the reasons for early opening and late closing of inlet and exhaust valves with reference to engine performance. | 5M | CO-1 | BL-2 |
|  | - Describe the construction, material selection, and function of piston rings. Explain the significance of ring gap and ring-to-groove clearance on engine performance. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | Explain the working of a four-stroke CI engine with a neat P-V diagram. Compare the theoretical Diesel cycle with the actual cycle, identifying the sources of deviation and their effects on thermal efficiency. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | - Describe the three stages of combustion in a petrol engine. Explain how engine speed and ignition timing affect the occurrence and intensity of knocking in SI engines. | 5M | CO-1 | BL-3 |
|  | - Determine the stoichiometric air requirement and percentage excess air for complete combustion of petrol (approximate formula C₈H₁₈). If the fuel mass flow rate is 0.5 kg/min, calculate the actual air mass flow rate for an AFR of 14.7:1. Also find the CO₂ produced per minute. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 4 | Describe the different types of combustion chambers used in SI engines (Hemispherical, Wedge, and Pentroof types). For each type, explain the flame travel path, thermal loading, and its susceptibility to detonation. Recommend the best design for a high-performance naturally aspirated engine with justification. | 10M | CO-1 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - With neat sketches, explain the construction and working principles of a hot-wire Mass Air Flow (MAF) sensor and a Manifold Absolute Pressure (MAP) sensor. State how their outputs differ under idle and wide-open-throttle conditions. | 5M | CO-2 | BL-2 |
|  | - Compare Throttle Body Injection (TBI) and Multi-Point Fuel Injection (MPFI) systems under the following criteria: (i) spray quality and fuel atomization, (ii) transient response during acceleration, (iii) cylinder-to-cylinder mixture distribution, and (iv) emission levels. | 5M | CO-2 | BL-4 |
| OR |  |  |  |  |
| 6 | Explain the working principle of a Gasoline Direct Injection (GDI) system with a neat block diagram. Describe the injection strategies employed during cold start, part load, and full load operating conditions. Explain how the ECU controls injection pressure, timing, and duration using sensor feedback. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | - Draw and explain the working of a pressurized water cooling system. Describe the function of the wax-type thermostat in controlling coolant flow and explain its impact on engine warm-up and fuel economy. | 5M | CO-3 | BL-2 |
|  | - An engine dissipates 18 kW of heat to the cooling system. Coolant water enters at 75°C and exits at 90°C. Specific heat of water = 4.18 kJ/(kg·K). (i) Calculate the mass flow rate of coolant required. (ii) If the engine runs at 3000 rpm (4-stroke, 4-cylinder), calculate the heat rejected per power stroke per cylinder. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | Explain the working of a turbocharger system with a neat diagram showing all components. Discuss: (i) the degree of supercharging and its effect on engine output power; (ii) intercooling and its thermodynamic benefits; (iii) turbo lag — its causes and methods to reduce it including variable geometry turbines. | 10M | CO-3 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - A four-cylinder, four-stroke petrol engine has the following specifications: bore = 80 mm, stroke = 100 mm, speed = 3000 rpm, indicated mean effective pressure = 7.0 bar, frictional power = 4.2 kW. Determine: (i) Indicated power, (ii) Brake power, (iii) Mechanical efficiency, and (iv) BMEP. | 5M | CO-4 | BL-3 |
|  | - Explain the concept and necessity of a Variable Compression Ratio (VCR) engine. Describe any two mechanisms used to achieve variable CR and state the operating range of CR used in such engines. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | Compare the working principles, combustion strategies, and emission characteristics of HCCI (Homogeneous Charge Compression Ignition), stratified charge, and lean burn engines. For each engine type: (i) describe the charge preparation method; (ii) identify the main advantage over a conventional SI engine; (iii) state the primary technical challenge limiting widespread adoption. | 10M | CO-4 | BL-4 |

****