| Subject Code 22PC1ME206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

DYNAMICS OF MACHINERY

(ME)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | State the purpose of Chebyshev spacing | 2M | CO- | BL- |
| --- | --- | --- | --- | --- |
|  | Define sensitiveness of a governor. | 2M | CO- | BL- |
|  | What is a block brake? | 2M | CO- | BL- |
|  | Define dynamic balancing. | 2M | CO- | BL- |
|  | Define critical or whipping speed of a shaft | 2M | CO- | BL- |

PART- B				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | The turbine rotor of a ship has a mass of 20 tonnes and a radius of gyration of 0.75 m. Its speed is 2000 r.p.m. The ship pitches 60 above and below the horizontal position. One complete oscillation takes 18 seconds and the motion is simple harmonic. Determine (a) the maximum couple tending to shear the holding down bolts of the turbine. (b) The maximum angular acceleration of the ship during pitching and (c) The direction in which the bow will tend to turn while rising, if the rotation of the rotor is clockwise when looking from rear | 10M | CO- | BL- |
| OR |  |  |  |  |
| 2 | - Explain pitching, rolling, and steering in ships due to gyroscopic effect | 4M | CO- | BL- |
|  | - An aircraft consists of a propeller. It also consists of engine and propeller mass moment of inertia 150 kg-m2. The engine rotates at 3600 rpm in a sense clockwise looking from rear. The aircraft completes half circle of radius 100 m towards left when flying at 360 km per hr. Determine the gyroscopic couple on the air-craft and state its effect. | 6M |  |  |
| UNIT-II |  |  |  |  |
| 3 | The equation of a turning moment curve at a three crank engine is 2500 +750 sin 30 N m where θ is the crank angle is radians. The mean speed of the engine is 240 rpm. The flywheel and other rotating parts attached to the engine have a mass of 500 kg at a radius of gyration of 1m calculate.(i) the power of the engine (ii) total fluctuation of the speed of the flywheel in percentage when the resisting torque is constant. | 10M | CO- | BL- |
| OR |  |  |  |  |
| 4 | In an engine governor of the Porter type, the upper and lower arms are 200 mm and 250 mm respectively and pivoted on the axis of rotation. The mass of the central load is 15 kg, the mass of each ball is 2 kg and friction of the sleeve together with the resistance of the operating gear is equal to a load of 25 N at the sleeve. If the limiting inclinations of the upper arms to the vertical are 300 and 400, find, taking friction into account, range of speed of the governor | 10M | CO- | BL- |
| UNIT-III |  |  |  |  |
| 5 | - A single dry plate clutch transmits 7.5 kW at 900 r.p.m. The axial pressure is limited to 0.07 N/mm2. If the coefficient of friction is 0.25, find Mean radius and face width of the friction lining assuming the ratio of the mean radius to the face width as 4, and Outer and inner radii of the clutch plate. | 7M | CO- | BL- |
|  | - Compare single plate, multi-plate, and cone clutches. | 3M | CO- | BL- |
| OR |  |  |  |  |
| 6 | A Simple band brake is applied to a drum of 560mm diameter, which rotates at 240 rpm. Angle of contact of band is 270o. One end of the band is fastened to a fixed pin and the other end of the brake lever 140mm from the fixed pin. The brake lever is 800mm long and is placed perpendicular to the diameter that bisects the angle of contact. The coefficient of friction is 0.3; determine the necessary pull at the end of the lever to stop the drum if 40 kW of power is being absorbed. Also find the width of the band if its thickness is 3mm and the maximum tensile stress is limited to 40N/mm2 | 10M | CO- | BL- |
| UNIT-IV |  |  |  |  |
| 7 | Each crank of a four cylinder vertical engine is 225 mm. The reciprocating masses of the first, second and the third cranks are 100 kg, 120 kg and 100 kg and the planes of rotation are 600 mm, 300 mm and 300 mm from the plane of rotation of the third crank. Determine the mass of the reciprocating parts of the third cylinder and the relative angular positions of the cranks if the engine is in complete primary balance | 5M | CO- | BL- |
| OR |  |  |  |  |
| 8 | A,B,C and D are four masses carried by a rotating shaft at radii 100 , 125, 200 and 150 mm respectively. The planes in which the masses revolve are spaced 600 mm apart and the masses of B ,C and D are 10 kg, 5 kg and 4 kg respectively. Find the required mass A and the relative angular settings of the four masses so that the shaft shall be in complete balance. | 10M | CO- | BL- |
| UNIT-V |  |  |  |  |
| 9 | A shaft 1.5 m long, supported in flexible bearings at the ends carries two wheels each of 50 kg mass. One wheel is situated at the centre of the shaft and the other at a distance of 375 mm from the centre towards left. The shaft is hollow of external diameter 75 mm and internal diameter 40 mm. The density of the shaft material is 7700 kg/m3 and its modulus of elasticity is 200 GN/m2. Find the lowest whirling speed of the shaft, taking into account the mass of the shaft. | 5M | CO- | BL- |
| OR |  |  |  |  |
| 10 | - Discuss the effect of inertia of the shaft in longitudinal and transverse vibrations | 3M | CO- | BL- |
|  | - A shaft 50 mm diameter and 3 metres long is simply supported at the ends and carries three loads of 1000 N, 1500 N and 750 N at 1 m, 2 m and 2.5 m from the left support. The Young's modulus for shaft material is 200 GN/m2. Find the frequency of transverse vibration | 7M |  |  |

****