| Subject Code 22ES1EE103 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular and Supplementary Examinations, May 2026

FUNDAMENTALS OF ELECTRICAL AND ELECTRONICS ENGINEERING

(ME)

Time: 3 hours                                                                        				Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A					5X2=10M

|  | State Kirchoff’s Current Law (KCL) with an example. | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the working principle of a DC motor and what way it is differ with DC generator. | 2M | CO-3 | BL-2 |
|  | List the different losses in a transformer. Mention how losses can influence the efficiency of a transformer? | 2M | CO-2 | BL-2 |
|  | Define slip. Why induction motor cannot run at synchronous speed? | 2M | CO-3 | BL-1 |
|  | What are the applications of SCR? | 2M | CO-4 | BL-1 |

PART- B				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Calculate the equivalent resistance between A&B using series and parallel combination networks. | 5M | CO-1 | BL-3 |
|  | - A single-phase AC circuit which consists of a capacitance of 5µF connected in series with 10 ohms resistor across a 230V, 50Hz supply. Calculate the capacitive reactance, impedance of the circuit and current flowing through the circuit. | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | For the signal given in figure (shown below), determine the following - Average value of the signal, ii) RMS value of the signal | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | - Derive the EMF equation of a DC generator. | 5M | CO-3 | BL-2 |
|  | - An 8 pole DC shunt generator with 778 wave connected conductors and running at 600 rpm. Supplies a load of 10 Ω at a terminal voltage of 250 V. The armature resistance is 0.2 Ω and shunt field resistance of 250 Ω. Find the armature current, induced EMF and flux per pole. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 4 | Discuss the armature control technique of a dc motor for speed control. A 220V DC shunt motor runs at 800 rpm and takes an armature current of 50A. What is the resistance to be added to the field circuit to increase the speed to 1000 rpm at an armature current of 80A, Ra = 0.15 Ω and Rf = 200 Ω. | 10M | CO-3 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | - Describe the principle of operation of a single phase transformer in detail. | 5M | CO-2 | BL-2 |
|  | - With a neat circuit diagram discuss the open circuit (OC) test on a single phase transformer. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 6 | Derive the expression for regulation of a transformer with a neat phasor diagram. A 10kVA, 2500/250V, 50Hz, single phase transformer gave the following test results: OC Test (on Low Voltage side): 250V, 0.8A, 50W; SC Test (on High Voltage side): 60V, 3A, 45W. Compute the voltage regulation at full load and 0.8p.f. leading. | 10M | CO-2 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | Discuss the “voltage regulation” of a three phase alternator in detail. Also discuss any of the method to calculate the voltage regulation of a synchronous generator. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 8 | Derive the expression for torque of a three phase induction motor. Also Explain its torque – slip characteristics in detail with a neat sketch. | 10M | CO-2 | BL-2 |
| UNIT-V |  |  |  |  |
| 9 | - Discuss the characteristics of Light Emitting Diode (LED). | 5M | CO-4 | BL-2 |
|  | - Write short notes on logic gates. | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | What is avalanche break down of a PN junction diode? Where it occurs in the V-I characteristics of PN junction? Explain in detail with a neat sketch. Also discuss the applications of it. | 10M | CO-4 | BL-2 |

****