| Subject Code 22PC1AE208 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

FLUID MECHANICS AND HEAT TRANSFER

(AE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | State the continuity equation for incompressible flow and explain its physical significance. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Water flows through a horizontal pipe of diameter 100 mm at 2 m/s. If the pipe diameter reduces to 50 mm, calculate the velocity at the reduced section (neglect losses). | 2M | CO-2 | BL-3 |
|  | A plane wall of thickness 0.2 m has thermal conductivity 0.5 W/m·K. If the temperature difference across the wall is 20°C, calculate the heat flux. | 2M | CO-3 | BL-3 |
|  | State the significance of Reynolds number in convective heat transfer. | 2M | CO-3 | BL-2 |
|  | State Stefan–Boltzmann law and write its mathematical expression. | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Calculate the capillary effect in millimeters a glass tube of 4 mm diameter, when immersed in (a) water (b) mercury. The temperature of the liquid is 200 oC and the values of the surface tension of water and mercury at 200 oC in contact with air are 0.073575 and 0.51 N/m respectively. The angle of contact for water is zero that for mercury 130o. Take specific weight of water as 9790 N / m3 | 10M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | If the velocity profile of a fluid over a plate is a parabolic with the vertex 202 cm from the plate, where the velocity is 120 cm/sec. Calculate the velocity gradients and shear stress at a distance of 0,10 and 20 cm from the plate, if the viscosity of the fluid is 8.5 poise. | 10M | CO-1 | BL-3 |
| UNIT-II |  |  |  |  |
| 3 | The rate of flow through a horizontal pipe is 0.25 m3/s. the diameter of the pipe which is 200 mm is suddenly enlarged to 400mm. the pressure intensity in the smaller pipe is 11.772 N/cm2. Determine (i). Loss of head due to sudden enlargement (ii). Pressure intensity in large pipe. (iii) Power lost due to enlargement. | 10M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Derive the expression for loss of head due to friction in pipes or Darcy – Weisbach Equation. | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |
| 5 | A gas filled tube has 2 mm inside diameter and 250 mm length. The gas is heated by an electrical wire of diameter 0.05 mm located along the axis of the tube. The current and voltage drop across the heating element are 0.5 A and 4V respectively. If the measure wire and inside wall temperatures are 175 °C and 150 °C respectively. Determine the thermal conductivity of gas filling the tube. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | A stainless steel fin (k=20W/mK) having a diameter of 20 mm and length of 0.1 m is attached to a wall at 300 °C. The ambient temperature is 50 °C and the heat transfer coefficient is 10W/m2K.The fin tip is insulated. Determine (a) The rate of heat dissipation from the fin (b) The temperature at the fin tip | 10M | CO-3 | BL-3 |
| UNIT-IV |  |  |  |  |
| 7 | During a cold winter day, wind at 55 km/h is blowing parallel to a 4 m high and 10 m long wall of a house. If the air outside is at 5°C and the surface temperature of the wall is 12°C, determine the rate of heat loss from that wall by convection. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 8 | The local atmospheric pressure in Denver, Colorado (elevation 1610 m), is 83.4 kPa. Air at this pressure and 20°C flows with a velocity of 8 m/s over a 1.5 m X 6 m flat plate whose temperature is 140°C. Determine the rate of heat transfer from the plate if the air flows parallel to the (a) 6-m-long side and (b) the 1.5-m side. | 10M | CO-3 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | Two concentric spheres of diameters D1 = 0.3 m and D2 = 0.4 m are maintained at uniform temperatures T1 = 700 K and T2 = 500 K and have emissivities ε1 = 0.5 and ε2 = 0.7, respectively. Determine the net rate of radiation heat transfer between the two spheres. Also, determine the convection heat transfer coefficient at the outer surface if both the surrounding medium and the surrounding surfaces are at 30 oC. Assume the emissivity of the outer surface is 0.35. | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 10 | Consider a cubical furnace of side 5 m, whose surfaces closely approximate black surfaces. The base, top, and side surfaces of the furnace are maintained at uniform temperatures of 800 K, 1500 K, and 500 K, respectively. Determine (a) the net rate of radiation heat transfer between the base and the side surfaces, (b) the net rate of radiation heat transfer between the base and the top surface, and (c) the net radiation heat transfer from the base surface. | 10M | CO-4 | BL-3 |

****