| Subject Code 22PC1ME210 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year I Semester Regular and Supplementary Examinations, May 2026

FLUID MECHANICS AND MACHINERY

(EEE)

Time: 3 hours                                                                        			 	Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | What is specific gravity? How it is related to density. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | Explain about hydraulic grade line. | 2M | CO-2 | BL-2 |
|  | What do mean by Impact of Jet. And write the equation for the force exerted by a jet on flat vertical plate. | 2M | CO-3 | BL-2 |
|  | Define cavitation. What are the problems occurs with cavitation? | 2M | CO-4 | BL-2 |
|  | Define Specific speed of a pump. | 2M | CO-4 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Define the surface tension, Prove that the relationship between surface tension and pressure inside a droplet of liquid given by P=4σ/d | 5M | CO-1 | BL-3 |
|  | - State and derive Newton’s Law of Viscosity | 5M | CO-1 | BL-2 |
| OR |  |  |  |  |
| 2 | The Stream function for a two dimensional flow is given by ѱ = 2xy, calculate the velocity at the point P (2, 3). Find the velocity potential function Φ. | 10M | CO-1 | BL-4 |
| UNIT-II |  |  |  |  |
| 3 | - State and derive Bernoulli’s equation. | 5M | CO-2 | BL-3 |
|  | - List minor losses of energy in a pipe flow and write the equations. | 5M | CO-2 | BL-2 |
| OR |  |  |  |  |
| 4 | The diameter of a pipe at the sections 1 and 2 are 10 cm and 15cm respectively. Find the discharge through the pipe if the velocity of water flowing through the pipe at section 1 is 5m/sec. Determine also the velocity at section 2. | 10M | CO-2 | BL-4 |
| UNIT-III |  |  |  |  |
| 5 | - Draw the layout of hydroelectric power plant and write about components in it. | 5M | CO-3 | BL-2 |
|  | - Derive the expression for force exerted by the jet when it strikes a stationary symmetrical curved plate at its center. | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |
| 6 | A 7.5cm diameter jet having a velocity of 30m/sec strike a flat plate the normal of which is inclined 35 degrees to the axis of jet. Find the normal force on the plate (i) when the plate is stationary and (ii) when the plate is moving with a velocity of 5 m/s. | 10M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |
| 7 | - Give the comparison between Francis and Kaplan turbines. | 5M | CO-4 | BL-2 |
|  | - Define unit quantities of turbine and derive the equations for them. | 5M | CO-4 | BL-3 |
| OR |  |  |  |  |
| 8 | A pelton wheel has a mean bucket speed 35m/sec with a jet of water flowing at the rate of 1m3/sec under a head of 270m. The bucket deflects the jet through an angle of 1700. Calculate the power delivered to the runner and the efficiency of head of the turbine. The co-efficient velocity cv=0.98. | 10M | CO-4 | BL-4 |
| UNIT-V |  |  |  |  |
| 9 | - Differentiate between single – stage and multistage centrifugal pumps. Describe multistage pump with i) impellers in parallel (ii) impellers in series. | 5M | CO-4 | BL-2 |
|  | - What are the different losses and efficiencies involved in the centrifugal pump? | 5M | CO-4 | BL-2 |
| OR |  |  |  |  |
| 10 | The following details refer to working of single acting reciprocating pump. Find the slip, coefficient of discharge and theoretical power required to drive the pump. Piston diameter=15cm, crank radius = 15cm, Dia of delivery pipe=10cm, Discharge of the pump=0.31m3/min, Total lift=15m. Speed of the pump=60rpm. | 10M | CO-4 | BL-42 |

****