| Subject Code 22PC1CE206 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

HYDRAULIC ENGINEERING AND HYDRAULIC

(CE)

Time: 3 hours                                                                          			 Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | For a rectangular channel with bed width 4 m, flow depth 1.8 m and Manning’s N = 0.022, calculate the Chezzy’s coefficient C. | 2M | CO-1 | BL-2 |
| --- | --- | --- | --- | --- |
|  | What is meant by Hydraulic Jump? | 2M | CO-2 | BL-1 |
|  | Find force exerted by jet of water of diameter 100 mm on a stationary flat plate, when strikes the plate normally with a velocity of 30 m/s. | 2M | CO-3 | BL-2 |
|  | What is draft tube? | 2M | CO-4 | BL-1 |
|  | What is the use of multistage centrifugal pump? | 2M | CO-4 | BL-1 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Prove that for the trapezoidal channel of most economical section. Half of top width = Length of one of the sloping sides. Hydraulic mean depth = 1/2 Depth of flow. | 10M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2 | What do you understand by open channel flow? Differentiate between (i) steady flow and unsteady flow (ii) Uniform flow and non-uniform flow (iii) Critical flow, sub- critical flow, super critical flow and (iv) Laminar and turbulent flow in an open channel. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | Explain the terms mild, critical, steep, horizontal and adverse slopes. | 10M | CO-2 | BL-2 |
| OR |  |  |  |  |  |
| 4 | Obtain an expression for the depth after the hydraulic jump and the loss of head due to the jump. Write the assumptions made. | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |  |
| 5 | A jet of water moving at 20 m/s impinges on a symmetrical curved vane shaped to deflect the jet through 120o . If the vane is moving at 5 m/s, find the angle of the jet so that there is no shock at inlet. Also determine the absolute velocity of exit in magnitude and direction, and the work done. | 10M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | Explain the procedure for solving problems by Buckingham’s ∏ theorem. | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |  |
| 7 | Design a Francis turbine runner with the following data. Net head = 68 m, Speed = 750 rpm, Output power = 330 kw, Hydraulic efficiency = 94%, Over all efficiency = 85%, Flow ratio = 0.15, Breadth ratio = 0.1, Inner dia. of runner is 0.5 outer dia. Also assume 6% of circumferential area of the runner to be occupied by the thickness of the vanes. Velocity of flow remains constant throughout and flow is radial at exit. | 10M | CO-4 | BL-4 |
| OR |  |  |  |  |  |
| 8 | Draw a neat diagram of Pelton turbine and explain its working. | 10M | CO-4 | BL-2 |
| UNIT-V |  |  |  |  |  |
| 9 | A single stage centrifugal pump working for a manometric head of 14m which delivers 144 litres of liquid per second. If the speed of rotation is 950 r.p.m. Calculate specific speed of the pump | 10M | CO-4 | BL-3 |
| OR |  |  |  |  |  |
| 10 | Draw the neat diagram of Centrifugal pump and explain its working. | 10M | CO-4 | BL-2 |

****