| Subject Code 22PC1EI204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

LINEAR IC APPLICATIONS

(EIE)

Time: 3 hours                                                                          		 	Max. Marks: 60 Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | Mention any two features of 741 op-amp? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | What is the need of an instrumentation amplifier? | 2M | CO-2 | BL-2 |
|  | Define the Butterworth filter? | 2M | CO-2 | BL-1 |
|  | List the applications of 565 PLL? | 2M | CO-3 | BL-1 |
|  | Describe AD 574 (12-bit ADC) and its specifications? | 2M | CO-5 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | - Draw and explain the block diagram of an op-amp? | 5M | CO-1 | BL-2 |
|  | - For a typical op-amp, ICQ=15 μAand C=35 pF. The peak value of input is 12 V. Determine slew rate and maximum possible frequency of input voltage that can be applied to get undistorted output? | 5M | CO-1 | BL-3 |
| OR |  |  |  |  |
| 2 | Explain DC coupling and cascade differential amplifier stages with neat diagrams? | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |
| 3 | - Explain inverting amplifier using op-amp with circuit diagram and gain derivation? | 5M | CO-2 | BL-2 |
|  | - Find the gain and output voltage for a non-inverting amplifier using op-amp when input voltage is (i) +0.5 V (ii) −3 V. Assume supply voltage employed is ±12 V, Rf=10 kΩ,R1=1 kΩ? | 5M | CO-2 | BL-3 |
| OR |  |  |  |  |
| 4 | Explain non-inverting amplifier using op-amp with circuit diagram and gain derivation? | 10M | CO-2 | BL-2 |
| UNIT-III |  |  |  |  |
| 5 | - Differentiate between inverting and non-inverting comparators? | 5M | CO-3 | BL-4 |
|  | - Describe the internal block diagram and working of IC 1496 multiplier? | 5M | CO-3 | BL-2 |
| OR |  |  |  |  |
| 6 | Draw the circuit diagram of an astable multivibrator to generate the output signal with frequency of 1 kHz and the duty cycle of 75%? | 10M | CO-3 | BL-2 |
| UNIT-IV |  |  |  |  |
| 7 | - Explain the functional diagram of IC 555 timer? | 5M | CO-4 | BL-2 |
|  | - With neat sketch, explain about principles and description of individual blocks of 565? | 5M | CO-4 | BL-4 |
| OR |  |  |  |  |
| 8 | Design a VCO using IC 566 for a specified frequency range. Show calculations for timing components? | 10M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |
| 9 | - Explain the process of conversion in a Successive approximation ADC, with necessary diagrams? | 5M | CO-5 | BL-2 |
|  | - An 8-bit ADC outputs all 1’s when Vi=5.1 V. Find its a) Resolution and b) Digital output when Vi=1.28 V? | 5M | CO-5 | BL-3 |
| OR |  |  |  |  |
| 10 | Explain the dual-slope ADC with neat diagram and waveforms. Discuss why it is suitable for digital voltmeters? | 10M | CO-5 | BL-3 |

****