| Subject Code 22PC1EE204 |  |  |  |  |  |  |  |  | R22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

VNR VIGNANA JYOTHI INSTITUTE OF ENGINEERING AND TECHNOLOGY

(AUTONOMOUS)

B.Tech. II Year II Semester Regular and Supplementary Examinations, May 2026

POWER SYSTEMS –I

(EEE)

Time: 3 hours                                                        					Max. Marks: 60

Answer ALL questions in PART-A.

Answer any ONE question from each unit in PART-B.

PART–A 				             5X2=10M

|  | List advantages of Economizer and Air pre-heater? | 2M | CO-1 | BL-1 |
| --- | --- | --- | --- | --- |
|  | Define and Explain skin effect. | 2M | CO-2 | BL-1 |
|  | - Define reflection coefficient? | 2M | CO-3 | BL-1 |
|  | Define string efficiency? | 2M | CO-4 | BL-1 |
|  | Explain radial distribution system? | 2M | CO-5 | BL-2 |

PART- B 				           5X10=50M

| UNIT-I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | - Demonstrate the Hydro Electric power plant with neat sketch. | 6M | CO-1 | BL-2 |
|  | - Explain merits and demerits of Nuclear power station? | 4M | CO-1 | BL-2 |
| OR |  |  |  |  |  |
| 2 | Explain in detail construction and operation of Thermal power plant. | 10M | CO-1 | BL-2 |
| UNIT-II |  |  |  |  |  |
| 3 | - Describe in detail about concept of Transposed lines? | 4M | CO-2 | BL-1 |
|  | - A 3-phase, 50 Hz transmission line is 120 km long and delivers 25 MW at 132 kV to a load at 0.8 pf lagging. Line constants per phase per km: R=0.15 Ω/km X=0.3 Ω/km, X = 0.3Ω/km Y=j3×10-6 S/km Using Nominal-π method, determine Sending-end voltage, current and Voltage regulation? | 6M | CO-2 | BL-4 |
| OR |  |  |  |  |  |
| 4 | Determine Transmission line parameters of long transmission line? | 10M | CO-2 | BL-3 |
| UNIT-III |  |  |  |  |  |
| 5 | - Explain travelling waves and Bewley diagram | 5M | CO-3 | BL-3 |
|  | - Derive expression for string efficiency? | 5M | CO-3 | BL-3 |
| OR |  |  |  |  |  |
| 6 | - Derive expression for Corona loss? | 4M | CO-3 | BL-2 |
|  | - A 3-phase, 132 kV, 60 Hz transmission line consists of 1.8 cm radius conductor spaced 1.8 meters apart in equilateral triangular formation. If the temperature is 46ºC and atmospheric pressure is 82 cm, Take mo = 0·85 calculate the corona loss per km of the line. | 6M | CO-3 | BL-4 |
| UNIT-IV |  |  |  |  |  |
| 7 | - Derive an expression for Sag when the supports are at equal heights. | 5M | CO-4 | BL-2 |
|  | - An overhead transmission line has a span of 300 m between two supports at equal height. The conductor weighs 1.2 kg/m and the maximum allowable tension is 2500 kg. Find Sag in the conductor and Height of conductor at mid-span if supports are 20 m high. | 5M | CO-4 | 3 |
| OR |  |  |  |  |  |
| 8 | - Explain different methods to improve the string efficiency across suspension type insulators. | 5M | CO-4 | 2 |
|  | - In a string of four suspension insulators, if the voltage across second and third units is 11kV and 15kV respectively find the total working voltage of the string. | 5M | CO-4 | BL-3 |
| UNIT-V |  |  |  |  |  |
| 9 | Classify and explain about Various Distribution systems? | 10M | CO-5 | BL-4 |
| OR |  |  |  |  |  |
| 10 | - List various Differences between AC and DC Distribution system? | 5M | CO-5 | BL-1 |
|  | - A 2-wire DC distributor AB is 800 m long and is fed from end A at 240V. Loads are tapped as follows 40 A at 200 m from A , 60 A at 500 m from A and 30 A at 800 m from A Resistance of each conductor = 0.05 Ω/km Find Voltage at each load point and Voltage drop at far end. | 5M | CO-5 | BL-4 |